| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Feedback Channel Design to Support Unpaired Multi-Carrier Configurations (15.2.8.2.5.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D2 |  |
| Abstract | The current multi-carrier operation defined in P802.16m/D2 is mostly based on the assumption on paired DL/UL channel and TDD mode. But 16m is supposed also to support FDD mode and possibly the unpaired DL/UL channel number. How to make this configuration works in 16m system will need to be specified in the standard. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Feedback Channel Design to Support Unpaired Multi-Carrier Configurations

I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng

MediaTek Inc.

- Introduction

When consider the FDD mode, the following secondary carrier assignment shown in Figure 1 may happen to one AMS. For this case, the feedback channel of the only available UL carrier will need to transmit the feedbacks for both of the DL carriers.

Figure 1 Unpaired carrier assignment example#1

On the other hand, the secondary carrier assignment as the way shown in Figure is also not impossible. This might happen if two of the DL carriers are partially configured carrier. For this case, we need to decide which UL carrier the MS will need to transmit the feedback for DL carrier.

Figure 2 Unpaired carrier assignment example#2

- Possible Resolution

There are two stage problems we need to discuss. The first stage is to decide whether these two examples are going to be supported in 16m. The second stage is to figure out how to resolve those problems.

For the example shown in Figure 1, the possible solution is having the carrier index within the fast feedback channel. Another way is to allocate feedback channel for different DL carrier feedback separately. For the example shown in Figure 2, a problem need to be clarified in advance is how the UL transmissions of those two carriers are allocated. The feedback relationship between the DL/UL may be specified during the carrier assignment stage.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#42, page#151 in P802.16m/D2]

15.2.8.2.5.2 Feedback Channel Configuration to Support Unpaired DL/UL Carrier Configuration

----------------------------------------------------------End of the Text-----------------------------------------------------------