| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Carrier Definition to Support both TDD and FDD Modes (15.2.8.1) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D2 |  |
| Abstract | The current standard text has a sentence states the carrier means a set of DL/UL carrier. This precludes the possible spectrum partition flexibility by system operator. This issue has not been clearly discussed in TGm, more clarification will be necessary. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Carrier Definition to Support both TDD and FDD Modes

I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng

MediaTek Inc.

- Introduction

There is a sentence in P802.16m/D2 to state “In FDD systems, a carrier consists of a DL channel and a UL channel.”. This sentence itself is very confusing, because the “carrier” was always used as a physical terminology but now we’re changing it as logical. In addition, it mandates the 16m FDD system always have to support symmetric carrier distribution, which prevent the flexibility for network operator to determine how they want to configure their spectrum resource. Even for 16m TDD system, the carrier may not always be paired because of partially configured carrier. Obviously, the current sentence in P802.16m/D2 really makes no sense.

Consider the example shown in Figure 1, the FDD operator take 20MHz paired spectrum, but it may want to configure the DL and UL carriers in different way for better utilization.

Figure 1 Example of carrier configuration for FDD

In order to indicate this possibility, the definition of carrier must be a “physical” carrier rather than a “logical” pair of physical carriers. This can make the current multi-carrier message design easier to support both TDD and FDD mode, because the carrier implied through the P802.16m/D2 is indeed the “physical” carrier rather than logical carrier.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#65, page#145 in P802.16m/D2]

In FDD systems, a carrier consists of a DL channel and a UL channel.

For both FDD and TDD mode, the carrier refers to a physical frequency channel and be associated with a physical carrier index. When supporting FDD mode, each downlink carrier and each uplink carrier will be separated specified in the MAC management message.

----------------------------------------------------------End of the Text-----------------------------------------------------------