| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Carrier Definition to Support both TDD and FDD Modes (15.2.8.1) |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D2 |  |
| Abstract | The current standard text has a sentence states the carrier means a set of DL/UL carrier. This precludes the possible spectrum partition flexibility by system operator. This issue has not been clearly discussed in TGm, more clarification will be necessary. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Carrier Definition to Support both TDD and FDD Modes

I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

<<Changes starts from Line 20 of page 146 of D2 Draft, Section 15.2.8.1  >>

A primary carrier is fully configured while a secondary carrier may be fully or partially configured depending on deployment scenarios. A secondary carrier for an AMS, if fully configured, may serve as primary carrier for other AMS's. Multiple AMSs, each with a different primary RF carrier may also share the same secondary carrier. The following multicarrier operation modes are identified, which may all or independently be supported:

- Primary Carrier Change: The most basic multicarrier mode in which a ABS changes the physical layer support for an AMS from one primary carrier to another carrier while maintaining all MAC related context and security association under a single MAC layer instantiation. Such change of primary carrier may be transparent to all layer above the MAC and the network.

- Multicarrier Aggregation: The multicarrier mode in which the AMS maintains its physical layer connection and monitors the control signaling on the primary carrier while processing data on the secondary carrier. The resource allocation to an AMS may span across a primary and multiple secondary RF carriers. Link adaptation feedback mechanisms should incorporate measurements relevant to both primary and secondary carriers. In this mode the system may assign secondary carriers to an AMS in the downlink and/or uplink asymmetrically based on system load (i.e., for static/dynamic load balancing), peak data rate, or QoS demand.

- Multicarrier Primary to Secondary Carrier Switching: The multicarrier mode in which the AMS switches its physical layer connection from the primary to the secondary carrier per ABS' instruction. The AMS connects with the secondary carrier for the specified time period and then returns to the primary carrier. When the AMS is connected to the secondary carrier, the AMS is not required to maintain its transmit or receive through the primary carrier. This mode is used for switching to partially configured carriers or fully configured carriers for downlink transmission only service.

<< Insert in line 5 of page 161 in D2 Draft to create a new section 15.2.8.2.11.4>>

15.2.8.2.11.4 Multicarrier Aggregation mode

In Multicarrier aggregation mode, the ABS activates one or more secondary carriers through AAI_MC-CMD message and may make concurrent resource allocation using A-MAP messages IEs on multiple active carriers, including primary carrier. The AMS in multicarrier aggregation mode may be assigned the same STID used across primary and secondary carriers. The AMS supporting Multicarrier aggregation mode shall monitor all active carriers and follow the resource allocations while in active mode. The operation of AMS in active, sleep and idle mode follows the procedures described in respective subsection of section 15.2.8.

[Adopt the following text modification started from line#65, page#145 in P802.16m/D2]

In FDD systems, a carrier consists of a DL channel and a UL channel.

For both FDD and TDD mode, the carrier refers to a physical frequency channel and be associated with a physical carrier index. When supporting FDD mode, each downlink carrier and each uplink carrier will be separated specified in the MAC management message.

[Adopt the following text modification started from line#65, page#145 in P802.16m/D2]

Physical carrier index is the index for the available carriers supported by an IEEE 802.16m network, which is sorted from lower frequency to higher frequency.

----------------------------------------------------------End of the Text-----------------------------------------------------------