| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Allocation of Additional PRUs from Guard Sub-carriers (15.3.3.5.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D2 |  |
| Abstract | The current P802.16m/D2 already includes basic text description to allow data transmission over guard sub-carriers to increase peak transmission rate and cell throughput. This contribution suggests some text enhancement to make the AWD description more complete to allow implementation on this feature. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Allocation of Additional PRUs from Guard Sub-carriers

I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng

MediaTek Inc.

- Introduction

The motivation to allow data transmission over guard sub-carriers is to increase the peak transmission rate the cell throughput by minimum overhead consumption and hardware complexity. The first assumption is that the transmission over guard sub-carriers will be allocated in unit of PRU, which has been agreed in SDD. Here further states the CRU will be used because there will be very limited PRUs available for data transmission (e.g. 3 PRUs in between for 2×10MHz carriers).

In addition, another assumption is that this feature should be enabled by minimum overhead and complexity cost. Because this is supposed to be an optional feature in WiMAX 2.0 system. In order to minimize the signaling overhead and because not every AMS may support this feature, it is suggested the transmission opportunity over guard sub-carriers in each frame will only be allocated to one AMS.

Another assumption is that only the AMS which supports both contiguous carriers (e.g. use 2048FFT to support 2×10MHz carriers) will utilize the guard sub-carriers in-between of those two carriers. So that the transmission or reception of the data over those guard sub-carriers can be easily done by post-FFT signaling processing with less requirements on RF filter and front-end re-design. This is also the reason to suggest transmission opportunity over those guard sub-carriers in between two carriers will be allocated together. The allocation through the A-AMAP in the carrier with lower frequency is just a straightforward idea, either way should be fine.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#xx, page#xx in P802.16m/D2]

15.3.3.5.3 Data Transmission over guard subcarriers in multi-carrier operation

When contiguous carriers are involved in multi-carrier operation, the guard sub-carriers between contiguous frequency channels and not overlapped with normal data sub-carriers may be utilized for data transmission. During the network entry procedure defined in 15.2.15 [TBD], AMS and ABS will negotiate their capability of supporting data transmission over guard sub-carriers through REG-REQ/RSP messages. Tthe ABS will notify the information on available guard sub-carriers eligible for data transmission to the AMS.

The transmission opportunity over the guard sub-carriers will be allocated in unit of Contiguous Resource Unit (CRU) as defined in 15.3.5.1.2. The transmission opportunity over the guard sub-carriers between the physical carrier i and physical carrier i+1 will be allocated through the A-AMAP transmitted over physical carrier i, where the center frequency of physical carrier i is lower than the one of physical carrier i+1. The transmission opportunity over guard sub-carriers allocated through each physical carrier will only be allocated to one AMS in each frame, where the corresponding A-AMAP will be transmitted in the first sub-frame.

----------------------------------------------------------End of the Text-----------------------------------------------------------