| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Unified Carrier Indication for AAI_NBR-ADV and AAI_MC-ADV Messages (15.2.3.11) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D2 |  |
| Abstract | Currently both the AAI_NBR-ADV and AAI_MC-ADV messages are requested to provide the frequency and bandwidth information of each carrier. In order to prevent unnecessary overhead, it is reasonable to allow AAI_MC-ADV provide global carrier information supported by system operator and the corresponding physical carrier index, so that AAI_NBR-ADV only need to broadcast the physical carrier indexes supported by each ABS. This can substantially reduce the broadcast overhead. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Unified Carrier Indication for AAI_NBR-ADV and AAI_MC-ADV Messages

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Paul Cheng

MediaTek Inc.

- Introduction

The IEEE 802.16m system will be most likely be deployed over multiple carriers, no matter so called “multi-carrier” operation is enabled or not. In order to indicate the center frequency and bandwidth of each carrier efficiently, the AAI_MC-ADV was adopted into P802.16m to utilize the physical carrier index to indicate the center frequency and the bandwidth of each carrier.

On the other hand, AAI_NBR-ADV also needs to provide the same information to facilitate the scanning operation by AMS. Compare with traditional method by separate parameters to indicate center frequency and bandwidth, utilizing the existing physical carrier index can obviously save overhead by single parameter. Since the number of ABS need to be indicated in AAI_NBR-ADV is large, the overhead reduction effect will be significant by utilizing physical carrier index in AAI_NBR-ADV.

In order to allow physical carrier index be used in AAI_NBR-ADV message, the content of AAI_MC-ADV message also need to be modified. The simplest way is to reuse the existing parameter definition but change the information scope from serving ABS to entire network. This means, the multi-carrier configuration indicated in AAI_MC-ADV message is all the carriers may be supported by the network. By letting AMS has this global information, the carriers supported by each ABS can be simply indicated by physical carrier index.

Therefore, the general operation flow is that AMS will need to receive AAI_MC-ADV message to obtain the global multi-carrier configuration once enter the network. Then it can further understand the carriers supported by neighboring ABSs through AAI_NBR-ADV message. This can save the overhead from AAI_NBR-ADV, which has been overloaded too much information inside.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#49, page#48 in P802.16m/D2]

15.2.3.11 AAI_NBR-ADV

…….

Each AAI_NBR-ADV message carries

- AAI_NBR-ADV change count

- number of total cell types

- segment information for this AAI_NBR-ADV message

- system information of ABSs from one or more cell types, which is of variable size.

For each cell type, shared configuration parameters are carried as follows

- FA information

- BW information

- Duplex mode

- MAC protocol version

For each fragment of the AAI_NBR-ADV, the fragment shall include the following parameters - Starting BS Index (i.e. this is the index offset from the last BS of the previous fragment. If this is first fragment, the Starting Index will be 0. Hence, each fragment has one Index which corresponds to the first BS in that fragment.)

Within each cell type, a list of ABSs are provided and the following parameters are carried for each ABS

- Index of neighbor ABS in the whole neighbor list maintained by serving ABS

- 48bit BS-ID

- ABS A-Preamble index

- indication whether full system information or partial information is carried for this ABS, which includes

  - SFH information

  - ESI information

  - Index to MC configuration,

  - Physical carrier index (6 bits, refer to the “Multi-carrier configuration index” defined in AAI_MC-ADV)

  - Number of MAC protocol versions supported (4 bits)

  - list of supported MAC protocol versions (4 bits per entry)

where for ABS of macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS(e.g., the serving BS or the first BS/ABS in this cell type); and for Wireless-MAN-OFDMA reference system, only 48-bit BS-ID and Preamble index are included in AAI_NBR-ADV.

where for macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS(e.g., the serving BS or the first BS/ABS in this cell type).

- SFH_encoding_format: Based on the present system information, the list of ABSs shall be categorized into three groups. The categorization is indicated by SFH_encoding_format parameter

- 0b00: Reserved

- 0b01: full Subpacket information

- 0b10: delta encoding with TBD Format

- 0b11: no SFH included

Table XXX—AAI_NBR-ADV message format

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 3 | NBR-ADV change count |
| Number of cell types | 3 |  |
| Cell type in this message | 3 |  |
| Total Number of Segments | TBD | Number of segments for this cell type |
| Segment Index | TBD | Segment index of this message in the specific cell type |
| BS number M | TBD | Total number of BSs to be included in this segment |
| Multi-carrier configuration change count | 4 | Refer to the change count in AAI_MC-ADV |
| for (i=0; i<M; i++) |  |  |
| { |  |  |
| Index of BS | 8 | The index of the BS in the whole AAI_NBR-ADV |
| BSID | 48 |  |
| Number of carriers (NC) |  |  |
| for( j=0; j<NC; j++) { |  |  |
| A-PREAMBLE index | TBD |  |
| A-PREAMBLE transmit power | 8 |  |
| Physical carrier index | 6 | Refer to the change count in AAI_MC-ADV |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode: >1 |
| For (i=0; i<P; i++) { |  |  |
| MAC protocol version | 4 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| } |  |  |
| } |  |  |
| SFH_encoding_format | 2 | 00: full Subpkt information 01: delta encoding with TBD format (the 1st BS in this cell type shall use full Subpkt encoding ) |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 1111. |
| If( SFH_encoding_format =00 |  | //encoding format type-1 |
| { |  |  |
| If( Control_bitmap[0] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt 1 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[1] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt2 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[2] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt3 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| } |  |  |
| If( SFH_encoding_format =01 |  |  |
| { |  |  |
| Delta information |  | Delta encoding, w.r.t. the reference BS |
| } |  |  |
| } | — | — |
| } | — | — |

[Adopt the following text modification started from line#56, page#148 in P802.16m/D2]

15.2.8.2.3.1.1 AAI_MC-ADV (Multi-Carrier Advertisement) Message

Table 725 AAI_MC-ADV MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| Multi-carrier configuration change count | 4 | A change count associated with a particular configuration for all possible carriers in the network. Incremented by 1 upon each update. |
| MC configuration broadcast flag | 1 | 0: only broadcast serving BS specific info 1: broadcast multi-carrier definition fro all available carriers in the network. |
| If (MC configuration broadcast flag == 1) { |  |  |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| Uniformity Flag | 1 | 0: All Carriers have the same Protocol Version, Carrier Type Duplexing Mode and Transmit Power. 1: otherwise |
| Physical Carrier Index of Current Carrier | 6 | The carrier that ABS broadcast this message |
| For (i=1; i<= Number of Carrier Groups; i++){ |  |  |
| Multi-Carrier Configuration Index | 6 | Index associated to Table 648 |
| Start Frequency Assignment Index | 6 | Frequency Assignment Index of the first carrier in carrier group #i |
| Number of Carriers | 4 | Number of carriers in carrier group #i |
| For (j=1; j <= Number of Carriers; j++) { |  |  |
| Physical Carrier Index | 6 |  |
| Preamble Index | 6 |  |
| If (Uniformity Flag == 1) ¦ ¦ (i == 1 && j == 1) ) { |  |  |
| Protocol Version | 2 | “00” IEEE 802.16-2009 only mode “01” IEEE802.16m and IEEE 802.16-2009 mixed mode “10” IEEE 802.16m only mode “11” Reserved |
| Carrier Type | 1 | “0” for fully configured “1” for partially configured |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Transmit Power | 6 | Maximum A-Preamble transmit power level |
| } |  |  |
| Physical Carrier Index of Current Carrier | 6 | The carrier that ABS broadcast this message |
| } |  |  |
| } |  |  |
| } |  |  |
| Serving BS Carrier Number | 6 | //start broadcasting serving BS specific info |
| For (i=0; i<Serving BS Carrier Number-1; i++){ |  |  |
| Physical Carrier Index | 6 |  |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode: >1 |
| For (i=0; i<P; i++) |  |  |
| { |  |  |
| MAC protocol version | 4 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m MAC version. |
| Preamble Index | 6 |  |
| } |  |  |
| } |  |  |

[Adopt the following text modification into line#3, page#284 in P802.16m/D2]

Table 764 Center frequency and bandwidth of each adjacent carriers within a carrier group (with sub-carrier alignment)

| Index | Multi-Carrier Configuration (MHz) | Frequency Offset Δf (kHz) |
| --- | --- | --- |
| 1 | {5} | {0} |
| 2 | {7} | {0} |
| 3 | {8.75} | {0} |
| 4 | {10} | {0} |
| 5 | {20} | {0} |
| 6 TBD | {10, 10} | {0, -3.1248} |

----------------------------------------------------------End of the Text-----------------------------------------------------------