| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the S-SFH SP1 IE Contents on the Ranging Channels in IEEE P802.16m/D2 (15.3.6.5.1.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Heejeong Cho and HyunWoo Lee LG Electronics Hwasun Yoo and Chiwoo Lim Samsung Electronics Co., Ltd. Xinrong Wang, Yang-Seok Choi, Yi Hsuan, Shantidev Mohanty Intel Corporation | Voice: +82-31-450-7902 E-mail: hj77cho@lge.com, camille@lge.com hwasun.yoo@samsung.com {xinrong.wang; yang-seok.choi ; yi.hsuan ; shantidev.mohanty}@intel.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30a P802.16m/D2 : 15.3.6.5.1.2 |  |
| Abstract | This contribution proposes the ranging parameters into S-SFH SP1 IE. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the S-SFH SP1 IE Contents on the Ranging Channels in IEEE P802.16m/D2 (15.3.6.5.1.2)

Heejeong Cho and HyunWoo Lee

LG Electronics

Hwasun Yoo and Chiwoo Lim

Samsung Electronics Co., Ltd.

Xinrong Wang, Yang-Seok Choi and Yi Hsuan

Intel Corporation

# 1. Introduction

Ranging channel structure and physical resource mapping are defined in the subclauses 15.3.9.1.4.1 and 15.3.9.2.4 of the IEEE 802.16m DRAFT Amendment [1]. By using S-SFH, an ABS should configure essential ranging parameters such as ranging preamble code partition information (table 877), ranging channel allocations (table 878) and so on. Most of the ranging parameters are already defined in the S-SFH SP 1 IE, but their terms are not consistent with those described in the above subclauses. Furthermore, there is a ranging channel formats (table 871) missing.

# Reference

[1] IEEE P802.16m/D2, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” October 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 15.3.6.5.1.2 of P802.16m/D2

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

Table 791 – S-SFH SP1 IE format

[ Remove the following fields in Table 791 (S-SFH SP1 IE format) : Total 16 bits (3+2+8+3) ]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Unsync ranging allocation interval channel information (ranging region periodicity( | 3 |  |
| Unsync ranging location in the frame (time and frequency) | 2 |  |
| RNG codes information | 8 |  |
| Ranging code subset/ partition configuration | 3 |  |

[ Insert the following fields in Table 791 (S-SFH SP1 IE format): Total 15 bits]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Ranging channel allocation periodicity | 2 | Indicates the periodicity of ranging channel allocation according to the Table 878. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Table 878. The range of values is 0 ≤ OSF ≤ 3. |
| If ( FDM-based UL PUSC zone support ) { |  | Indicates the WirelessMAN-OFDMA support in the UL with FDM mode, when the Frame Configuration Index in S-SFH SP2 is equal to - 5 or 7 or 9 for 5/10/20MHz channel bandwidth according to Table 765 - 4 or 6 or 8 for 8.75MHz channel bandwidth according to Table 766 - 3 (with CP=1/8) for 7MHz channel bandwidth according to Table 767. |
| Start code information of ranging channel | 4 | Indicates the kns which is the parameter for start of code group (S). S= 16×kns+1 The range of values is 0 ≤ kns ≤ 15. |
| Ranging preamble code partition information | 4 | Indicates the number of initial, handover and periodic codes (N, O and M) according to the Table [TBD]. |
| Ranging channel location in the frequency domain | 3 | Indicate the start distributed LRU index for ranging channel. |
| } else { |  |  |
| Start code information of ranging channel for non-synchronized AMSs | 4 | Indicates the kns which is the parameter controlling the start root index of ranging preamble codes (rns0). rns0(kns)=4×kns+1 for ranging channel format 0. rns0(kns)=16×kns+1 for ranging channel format 1. The range of values is 0 ≤ kns ≤ 15. |
| Ranging preamble code partition information for non-synchronized AMSs | 4 | Indicates the number of initial and handover ranging preamble codes (NIN and NHO) according to the Table 877. |
| Number of cyclic shifted ranging preamble codes per root index for non-synchronized AMSs | 2 | Indicates the number of cyclic shifted codes per root index (Mns) for ranging preamble codes according to the Table 876. |
| Ranging channel formats for non-synchronized AMSs | 1 | Indicates the ranging channel formats number of Table 871. |
| } |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------