| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI_SCN-REQ/RSP/REP for Multi-Carrier Support (15.2.3.12) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D2 |  |
| Abstract |  |  |
| Purpose | For TGm discussion and adoption into P802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI_SCN-REQ/RSP/REP for Multi-Carrier Support

I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Paul Cheng

MediaTek Inc.

- Introduction

It is most likely that the IEEE 802.16m system will be deployed over multiple carriers. This implies many of the ABSs are actually multi-carrier ABS even though they only support single carrier operation for each AMS. Therefore, the scanning request, response and report message need to include physical carrier index to allow AMS report the scanning result for different carriers.

The current text in P802.16m/D2 already includes preliminary description on the carrier index in AAI_SCN-REQ/RSP message, but some minor details may need to be clarified. In addition, the carrier index has not been clearly described in AAI_SCN-REP message and need further amendment to the standard.

On the other hand, there is also some concern on current text to restrict only the logical carrier index can be used for the scanning on serving ABS. But this will result in a bug that the AMS will never scan a carrier which is not assigned, while the system may certainly have possibility want AMS switch to the carrier which was not previously assigned (e.g. from 2.3GHz to 700MHz, like switching between GSM900MHz and 1800MHz). The possibility to have this scenario is hard to judge, but it will be premature to preclude this possibility at such early stage. So that all the “carrier” in AAI_SCN-REQ/RSP/REP shall refer to physical carrier index rather than logical carrier index.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#60, page#50 in P802.16m/D2]

| N_Recommended_Carrier_Index | Number of carriers to be scanned at each neighboring ABS. | May be included when N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| --- | --- | --- |
| Recommended_Carrier_Index | Recommended physical carrier index for scan at each neighboring ABS. | Shall be included when N_Recommended_Carrier_Index > 0 |
| N_Recommended_Carrier_Index_at_Serving_ABS | Number of logical physical carrier index of the serving ABS to be scanned |  |
| Carrier index at serving ABS | Recommended logical physical carrier index to be used for scanning | Shall be included when N_Recommended_Carrier_Index_at_Serving_ABS > 0 |

[Adopt the following text modification started from line#17, page#53 in P802.16m/D2]

| N_Recommended_Carrier_Index | Recommended physical carrier index for scan at each neighboring ABS. | May be included for a multi-carrier target ABS |
| --- | --- | --- |
| Recommended_Carrier_Index | Recommended physical carrier index for scan at each neighboring ABS. | Included when N_Recommended_Carrier_Index > 0 |
| Carrier index at serving ABS | Recommended logical physical carrier index to be used for scanning | May be included when the serving ABS is a multi-carrier ABS |

[Adopt the following text modification started from line#35, page#55 in P802.16m/D2]

| Scanning report of each recommended physical carrier at each neighboring ABS |  | May be included for multi-carrier target ABSs |
| --- | --- | --- |

----------------------------------------------------------End of the Text-----------------------------------------------------------