| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the CDMA Allocation A-MAP IE (D2 - 15.2.11.2) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Youngkyo Baek Jaehyuk Jang Yeongmoon Son Jungje Son Samsung Electronics Heejeong Cho, HyunWoo Lee Youngsoo Yuk Kiseon Ryu Ronny (Yongho) Kim Jin Sam Kwa LG Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.15.3.1” |  |
| Abstract | This contribution proposes clarification on fragmentation of AAI_RNG-REQ message to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the CDMA Allocation A-MAP IE (D2 - 15.2.11.2)

Youngkyo Baek, Jaehyuk Jang,Yeongmoon Son, Jungje Son

Samsung Electronics

Heejeong Cho, HyunWoo Lee, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

# 1. Introduction

All unicast management messages can be fragmented. But, in case that any STID is not assigned yet to AMS, if AMS sends a management message (e.g. AAI_RNG-REQ) with fragmentation, there is no mention about how ABS can recognize the fragments and reassemble them. We need to clarify it. This issue is related to transmit ranging messages only because if a STID is assigned to AMS already, then the receiver can use the STID for MS identification.

As a remedy, we can use a RAID. That is, ABS allocates UL bandwidth by CDMA_Allocation_AMAP IE based on the same RAID if it receives UL bandwidth request (e.g. either BR without STID header or the fragment of the AAI_RNG-REQ message with PBREH) with a fragement.

However, currently RAID can cover 16 frames only, which is not enough because RAID contains only 4bit-length frame number index.

Hence we suggest enlength the size of RA-ID as follows.

RA-ID = (superframe number LSB4bits | frame_index 2bits | ranging_code_index 6bits | opportunity_index 2bits) and masking indicator be 2bit-length.

# 2. Text Proposal

Modify the sentences as follows (Page 178 line 56)

======================== Start of Proposed Text #1=====================

15.2.11.2 Grant

In the bandwidth request procedure, the grant for BW-REQ message is allocated by CDMA Allocation A-MAP IE, which is shown in Table 734. CDMA Allocation A-MAP IE is used for allocation of bandwidth to a user that requested bandwidth using a ranging code or BR code. AMS decodes the IE and checks the MCRC field by its specific 124-bit RA-ID and 24-bit Masking Indicator. The RA-ID is derived from calculated by a hash function with the AMS' random access attributes (i.e., superframe number (LSB 4bits), frame_number index (42 bits), ranging code/BR code index (6 bits) and opportunity index (2 bits)) as defined below:

RA-ID = (LSB 4bits of superframe number | frame_index | ranging_code_index | opportunity_index)

Masking indicator indicates the identifier used for CRC masking, as shown in Table 735

……….

Table 735—Description of the Masking Indicator

| Masking Indicator | Description |
| --- | --- |
| 0b0000 | MCRC is masked by 12-bit STID |
| 0b0001 | MCRC is masked by 1214-bit RAID for Ranging code |
| 0b0010 | MCRC is masked by 1214-bit RAID for bandwidth request code |

============================== End of Proposed Text#1 ===============

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010R2, “IEEE 802.16m Amendment Working Document”