| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal on Common Feedback Channel for E-MBS (15.3.9.1.6) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Chih-Yuan Lin, Pei-Kai Liao, Yih-Shen Chen MediaTek | chihyuan.lin@mediatek.com, |
| Re: | Letter Ballot #30a on the Draft Amendment (IEEE P802.16m/D2) |  |
| Abstract | Propose to include a common feedback channel for E-MBS |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Common Feedback Channel for E-MBS

Chih-Yuan Lin, Pei-Kai Liao and Yih-Shen Chen

MediaTek

---------------------------------------------- Start of Proposed Text ------------------------------------

<Please add following text in line 6 on page 479>

15.8.3.3 E-MBS common feedback channel

E-MBS may employ a common uplink channel which is used by AMSs to transmit feedback. If a predefined feedback condition is met, a predefined signal is transmitted through the common E-MBS feedback channel. With the feedback information, BS may perform link adaptation or QoS adaptation such as retransmission or source coding adaptation. The usage of the feedback channel by the ABS is out of scope.

---------------------------------------------- End of Proposed Text -----------------------------------------