| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Ranging Channel Structure for Synchronized AMSs in IEEE P802.16m/D2 (15.3.9.1.4.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Pei-Kai Liao, Yih-Shen Chen, Paul Cheng MediaTek Inc Yan-Xiu Zheng ITRI | pk.liao@mediatek.com zhengyanxiu@itri.org.tw |
| Re: | LB30a on IEEE P802.16m/D2 (15.3.9.1.4.2) |  |
| Abstract | The contribution proposes the text of synchronized ranging channel structure to be included in IEEE P802.16m/D2. |  |
| Purpose | To be discussed and adopted by TGm for IEEE P802.16m/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Ranging Channel Structure for Synchronized AMSs in IEEE P802.16m/D2

Pei-Kai Liao, Yih-Shen Chen, Paul Cheng

MediaTek Inc.

Yan-Xiu Zheng

ITRI

Introduction

In current D2, the description of ranging channel structure for synchronized AMSs is still not complete. This contribution is to propose detailed design to for TGm’s adoption to complete the section.

- Text Proposal

[Add the following into IEEE P802.16m/D2]

---------------------------------------------------------Start of the Text---------------------------------------------------------

# 15.3.9.1.4.2  Ranging channel structure for synchronized AMSs

The ranging channel for synchronized AMSs is used for periodic ranging. The AMSs that are already synchronized to the target ABS are allowed to transmit the periodic ranging signal. ForIn a femtocell, the ranging channel for synchronized AMSs shall perform can be used for initial ranging, handover ranging, and periodic ranging by the ranging channel for synchronized AMSs.

The physical structure in the ranging channel for synchronized AMSs occupies 72 subcarriers by K OFDMA symbol time. Table X-1 contains the formats and parameters of the ranging channel for synchronized AMSs.

Figure 551552 illustrates the basic unitphysical structures of ranging channel structure in the time domain where Tg and Tb are defined in <<<15.3.2.4.>>>, where XP(k), XP(k+N1), XP(k+N2) represent the starting code digit in each OFDM symbol and N2 > N1. The detailed code sequence mapping is described in <<15.3.9.2.4.2>>.

Table X-1—Formats and parameters of ranging channel for synchronized AMSs

| Format No. | Number of Symbols (K) | Description |
| --- | --- | --- |
| 0 | 3 | 72 subcarriers × 3 OFDM symbols per ranging channel allocation There are two ranging opportunities in one subframe. |
| 1 | 6 | Two duplicate 72 subcarriers × 3 OFDM symbols per ranging channel allocation There is one ranging opportunities in one subframe. |

- Format 0

- Format 1

Figure 552 – Ranging channel structure for synchronized AMSs in the time domain

Power control operation described in <<<15.3.9.4.3>>> applies to ranging signal transmission.

-----------------------------------------------------------End of the Text--------------------------------------------------------