| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Ranging Channel Code Sequences and Configurations for Synchronized AMSs in IEEE P802.16m/D2 (15.3.9.2.4.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Pei-Kai Liao, Yih-Shen Chen, Paul Cheng MediaTek Inc Yan-Xiu Zheng ITRI | pk.liao@mediatek.com zhengyanxiu@itri.org.tw |
| Re: | LB30a on IEEE P802.16m/D2 |  |
| Abstract | The contribution proposes the text of synchronized ranging channel structure to be included in IEEE P802.16m/D2. |  |
| Purpose | To be discussed and adopted by TGm for IEEE P802.16m/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Ranging Channel Code Sequences and Configurations for Synchronized AMSs in IEEE P802.16m/D2

Pei-Kai Liao, Yih-Shen Chen, Paul Cheng

MediaTek Inc.

Yan-Xiu Zheng

ITRI

Introduction

In current D2, the description of ranging channel code sequences and configurations for synchronized AMSs is still not complete. This contribution is to propose detailed design to for TGm’s adoption to complete the section.

- Text Proposal

[Add the following into IEEE P802.16m/D2]

---------------------------------------------------------Start of the Text---------------------------------------------------------

15.3.9.2.4.2 Ranging channel for synchronized AMSs

The Padded Zadoff-Chu codes with cyclic shifts are used for the ranging preamble basic codes. The pthranging preamble code xn(n,k) is defined by

(266)

where

p	is the index for pth ranging preamble code for nth OFDMA symbols within a basic unit which is made as the spth cyclic shifted sequence from the root index rp of Zadoff-Chu sequence.

The start root index, rs0, is broadcasted. NTOTAL is the number of periodic ranging preamble codes per sector (0~NPE -1) which is defined by Table (t.1). For femtocell, NTOTAL is the total number of initial (0~NIN -1), handover (NIN ~NIN +NHO -1) and periodic ranging preamble codes (NIN +NHO ~ NIN +NHO+NPE -1) per sector which is defined by Table 738 and Table (t.1).

m	is the unit of cyclic shift.

NCS     is the unit of cyclic shift according to the cell size. It is defined by , where is the number of cyclic shifted codes per Padded Zadoff-Chu root index according to Table X-2.

NRP	is the length of ranging preamble codes, which is equal to 214.

Cq(n)	is [TBD].

The start root index of ZC code (rs0), and the ranging preamble code information are broadcasted. The ranging preamble code information indicates the number of periodic ranging preamble codes. Figure X-1 shows the physical code sequence mapping of the ranging channel for synchronized AMSs.

Figure X-1—Physical code sequence mapping

Ranging channel configurations

The information of ranging channel allocation consists of the ranging configuration with AAI subframe-offset (OSF) for ranging resource allocation in the time domain where OSF is same AAI subframe-offset of ranging channel for non-synchronized AMSs defined in 15.3.9.2.4.1.

Table 879 shows the information of ranging channel allocation when number of UL AAI subframe per frame (NUL) is larger than 1.

Table 879—Ranging channel allocations for synchronized AMSs

| Configurations | The subframe allocating Ranging channel |
| --- | --- |
| 0 | mod(OSF+1, NUL)th UL subframe in every frame |
| 1 | mod(OSF+1, NUL)th UL subframes in the first frame in every superframe |
| 2 | mod(OSF+1, NUL)th UL subframe in the first frame in every [TBD]th superframe. |
| 3 | mod(OSF+1, NUL)th UL subframe of the first frame in every [TBD]th superframe. |

Table X-1 shows the number of cyclic shifted codes per root index.

Table X-1—The number of cyclic shifted codes per Padded Zadoff-Chu root index, Ms

| Index | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Ms | 1 | 2 | 4 | 8 |

----------------------------------------------------------End of the Text---------------------------------------------------------