| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dynamic Ranging Channel Allocation for 802.16m (15.3.6.5.2.15) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Yih-Shen Chen Kelvin Chou Pei-Kai Liao MediaTek Inc. | Yihshen.Chen@mediatek.com Peikai.Liao@mediatek.com Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30a on the Draft Amendment (IEEE P802.16m/D2) |  |
| Abstract | This contribution proposes AMS initialization procedure |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dynamic Ranging Channel Allocation for 802.16m

Yih-Shen Chen, Kelvin Chou, Pei-Kai Liao

MediaTek Inc.

- Introduction

In current IEEE 802.16m specification [1], the channel allocation information for initial/handover ranging and bandwidth request ranging are defined in SP1 IE and SP3 IE of SFH, respectively. An AMS who intends to perform the ranging operations shall obtain the SFH information first. Since the information is transmitted over SFH, the ranging channel allocation is semi-static; that is, the ranging channels are allocated in a pre-defined region until SFH is updated. The channel pre-allocation method can save broadcasting overhead; however, it would perform poorly when request arrival rate varies quickly.

In some scenarios, large handover or uplink bandwidth requests occur abruptly. For example: in rush hour, MSs move toward the same direction and lots of handover occurs. Another possible example is that large short-message texts flood out suddenly in New Year’s Eve. If those requests continue for a long time, SFH updating for ranging channel reconfiguration is a possible solution. However, if not, frequent SFH updating may interrupt normal operations and nullify some MAC broadcasting information, such as: AAI_NBR-ADV.

Actually, it is also stated in section 15.3.9.2.4.1 that

“….The ranging channel for handover ranging can also be allocated by AMAP based on ABS scheduling decision

in any AAI subframe, except the AAI subframe that has already been used for a regular allocation…”

Therefore, the concept of dynamic allocation of handover ranging channel is already in D2. In this contribution, we propose a new A-MAP IE for dynamic ranging channel allocation, which can be applied to unsynchronous ranging, synchronous ranging and bandwidth request ranging.

Proposed text:

Insert the following text into section 15.3.6.5.2.15

-------------------------------------------------------------------- Start of text-----------------------------------------------------------------------------

15.3.6.5.2.15 Ranging Channel Allocation A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP() { |  |  |
| A-MAP IE Type | 4 | Ranging Channel Allocation A-MAP IE |
| Ranging Channel Type | 2 | 0b00: Unsynchronized ranging channel 0b01: Synchronized ranging channel 0b10: BR ranging channel 0b11: Reserved |
| Number of Ranging Opportunities (N) | 2 |  |
| For(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Suband Index | 5 |  |
| } |  |  |
| Reserved |  |  |
| } |  |  |

--------------------------------------------------------------------End of text------------------------------------------------------------------------------

Reference

[1] IEEE 802.16m/D2, Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface