| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for Advanced Air Interface in LZone (15.2.6.4) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30a on the Draft Amendment (IEEE P802.16m/D2) |  |
| Abstract | This contribution proposes a new section to collect all the modification to the legacy system for AAI support |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support for Advanced Air Interface in LZone (15.2.6.4)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Introduction

There are several modifications to the Rev2 messages to be sent in LZone of a 16e/m coexisted ABS, e.g. FCH and Zone Switch TLV. This contribution proposes a new section to include all those modifications.

- Proposed Text

Add the following section in P802.16m/D2, page 592, line 31:
------------------------------------------------------------------- Text Start ---------------------------------------------------------------------------

15.10 Support for Advanced Air Interface in LZone

15.10.1 Support for network topology advertisement

15.10.1.1 DL frame prefix

Table XX1 defines the structure of DL_Frame_Prefix to be transmitted in LZone of an ABS supporting WirelessMAN-OFDMA Reference System.

Table XX1 OFDMA DL Frame Prefix format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL_Frame_Prefix_Format() { |  |  |
| Used subchannel bitmap | 6 | Bit #0: Subchannel group 0 Bit #1: Subchannel group 1 Bit #2: Subchannel group 2 Bit #3: Subchannel group 3 Bit #4: Subchannel group 4 Bit #5: Subchannel group 5 |
| 16e/16m coexistence indication | 1 | 0b0: WirelessMAN OFDMA Reference System only 0b1: WirelessMAN OFDMA Reference/Advanced System coexistence |
| Repetition_Coding_Indication |  | 0b00: No repetition coding on DL-MAP 0b01: Repetition coding of 2 used on DL-MAP 0b10: Repetition coding of 4 used on DL-MAP 0b11: Repetition coding of 6 used on DL-MAP |
| Coding_Indication |  | 0b000: CC encoding used on DL-MAP 0b001: BTC encoding used on DL-MAP 0b010: CTC encoding used on DL-MAP 0b011: ZT CC encoding used on DL-MAP 0b100: CC encoding with optional interleaver 0b101: LDPC encoding used on DL-MAP 0b110 to 0b111: Reserved |
| DL-Map_Length | 8 | — |
| Reserved | 4 | Shall be set to zero |
| } |  | — |

15.10.2 Support for zone switch operation

15.10.2.1 RNG-RSP management message encodings

The encodings in Table XX2 are specific to the RNG-RSP message sent in LZone of an ABS supporting WirelessMAN-OFDMA Reference System.

Table XX2 --- OFDMA-specific RNG-RSP message encodings

| Name | Type (1 byte) | Length | Value |
| --- | --- | --- | --- |
| MZone A-Preamble index | 41 | 1 |  |
| Time offset | 42 | 1 | Time offset |
| Action Time | 43 | 1 | Action time of zone switch from LZone to MZone. AMS performs zone switch at Action Time. If HO_Reentry_Mode=0, ABS stops all resource allocation for the AMS at LZone. |
| Zone Switch Mode | 44 | 1 | 0x01: AMS maintains its data communication with the ABS in LZone while performing network reentry in MZone; 0x00: AMS breaks data communication in LZone before performing network reentry in MZone. |
| NONCE_BS | 45 | 4 | For AMSID* derivation. PMK is shared between LZone and MZone during zone switch. |
| Temporary STID | 46 | 1 | Temporary STID for being used in MZone |
| Ranging initiation deadline | 47 | 1 | Valid time for Temporary STID. Shall be included if Temporary STID is included. |

------------------------------------------------------------------- Text End ----------------------------------------------------------------------------

Modify the following text in P802.16m/D2, page 132, line 32:
------------------------------------------------------------------- Text Start ---------------------------------------------------------------------------

An ABS uses one reserved bit in FCH to indicate the presence of its MZone, as defined in 15.10.1.1

------------------------------------------------------------------- Text End ----------------------------------------------------------------------------

Modify the following text in P802.16m/D2, page 134, line 37:
------------------------------------------------------------------- Text Start ---------------------------------------------------------------------------

The Zone Switch TLV shall include the following: parameters shall include the followings and are coded as TLV tuples as defined in 15.10.2.1

•MZone A-Preamble index.

•Time offset between LZone and MZone

•Action Time: Action time of zone switch from LZone to MZone. AMS performs zone switch at Action Time. If HO_Reentry_Mode=0, ABS stops all resource allocation for the AMS at LZone.

•Zone Switch Mode: If set to 1, the AMS maintains its data communication with the ABS in LZone while performing network reentry in MZone; otherwise it breaks data communication in LZone before performing network reentry in MZone.

•NONCE_BS: to derive AMSID*. PMK is shared between LZone and MZone during zone switch.

The Zone Switch TLV may also include the following:

•Temporary STID for being used in MZone

—Ranging initiation deadline: Valid time for Temporary STID. Shall be included if Temporary STID is included.

------------------------------------------------------------------- Text End ----------------------------------------------------------------------------