| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the Random Access ID (15.2.11.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30a on the Draft Amendment (IEEE P802.16m/D2) |  |
| Abstract | This contribution proposes modification to the RAID used in the CDMA Allocation A-MAP IE |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the Random Access ID (15.2.11.2)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Introduction

The 12-bit Random Access ID (RA-ID) has been defined as the concatenations of 4-bit frame index, 6-bit ranging code/BR code index and 2-bit opportunity index. However, this definition of RA-ID identifies only does not accommodate those ranging opportunities dynamically allocated using A-MAPs. In this contribution, we proposed modification to the derivation of RA-ID to address this issue. Out proposal is to identify the ranging opportunity using 4 attributes: frame number index (4 bits), ranging code/BR code index (6 bits), subframe index (3 bits), and subband index (5 bits). The RA-ID is then calculated by applying exclusive-OR on the ranging code index and the concatenation of frame index, subframe index and subband index, as illustrated below:

- Proposed Text

Apply the following text modification to P802.16m/D2, page 178, line 56 as follows:
--------------------------------------------------------- Text Start ---------------------------------------------------------------

15.2.11.2 Grant

In the bandwidth request procedure, the grant for BW-REQ message is allocated by CDMA Allocation A-MAP IE, which is shown in Table 734. CDMA Allocation A-MAP IE is used for allocation of bandwidth to a user that requested bandwidth using a ranging code or BR code. AMS decodes the IE and checks the MCRC field by its specific 12-bit RA-ID and 4-bit Masking Indicator. The RA-ID is calculated by a hash function with the AMS' random access attributes (frame number index (4 bits), ranging code/BR code index (6 bits), subframe index (3 bits), and subband index (5 bits) opportunity index (2 bits)) as defined below:

RA-ID = (frame_index | ranging code_index | opportunity_index)

RA-ID = ( (frame_index | subframe_index | subband index) ⊕ (ranging code_index | ranging code_index) )

where

a⊕b denotes the result of applying bit-wise exclusive-OR operation to bit strings a and b.

a|b denotes the result of concatenating bit strings a and b.

---------------------------------------------------------- Text End ---------------------------------------------------------------

- Appendix – Analysis of the collision probability

Simulation of the RAID collision probability has been conducted for up to N (N<=32) concurrent users within 80 milliseconds (24 frames). The simulation assumes the extreme case in which the CDMA Allocation A-MAP IEs for the N users are delayed by the ABS for 80 milliseconds and are sent concurrently at the very end (the 16th frame after the ranging channel in which the users sent their ranging codes). Ranging code index, frame index, subframe index and subband index of each user are generated using rand() in Matlab.