| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the AAI_RNG-ACK Message Format (15.2.3.3) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. Kevin Power , Yanling Lu, Masato Okuda Fujitsu | Kelvin.Chou@mediatek.com Kevin.power@uk.fujitsu.com |
| Re: | Letter Ballot #30a on the Draft Amendment (IEEE P802.16m/D2) |  |
| Abstract | Proposed text on the AAI_RNG-ACK message format |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on the AAI_RNG-ACK Message Format (15.2.3.3)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Kevin Power , Yanling Lu, Masato Okuda

Fujitsu

Introduction

The proposed parameters for the AAI_RNG-ACK message was adopted during Session#62 in San Francisco through AWD comment #268 (contribution C802.16m-09/1487r1 Remedy #2) but was not implemented in D1. This issue was raised through LB30 comment #301 during Session#63 in Jeju and was accepted, but again was not implemented in current D2. In the contribution, the parameters for the AAI_RNG-ACK are proposed again in a tabularized format.

Proposed Text

Remedy 1

In the P802.16m/D2, page 39, line 41, insert the following subsection before the existing 15.2.3.3 and increase the section numbers thereafter accordingly:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

15.2.3.3 AAI_RNG-ACK

The AAI_RNG-ACK message is sent by the ABS in response to the CDMA ranging request during initial ranging, period ranging and HO ranging in order to provide PHY-level adjustment (e.g. timing offset, power level and frequency offset). Upon reception of ranging codes, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE using broadcast STID. When the AAI_RNG-ACK message is sent unsolicitedly to specific user, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE with unicast STID. If all the ranging statuses of the detected ranging preambles are equal to ‘success’, the AAI_RNG-ACK may be omitted. The AAI_RNG-ACK shall include the following parameters:

Table XXX --- Parameters for AAI_RNG-ACK

| Name | Value | Description |
| --- | --- | --- |
| Management Message Type | AAI_RNG-ACK |  |
| RNG-ACK Bitmap | The size of this bitmap is decided by the number of ranging slots in the previous frame. Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. |  |
| Number of received codes |  | The number of ranging code indices detected in this corresponding ranging opportunity. Shall be included for each bit=0b1 in the RNG-ACK Bitmap |
| Code Index |  | Code index received in this ranging opportunity. Shall be included for each received ranging codes if Number of received codes > 0 |
| Ranging status | Used to indicated whether UL messages are received within acceptable limits by the ABS 0b01: continue 0b11: abort 0b00: success | Shall be included for each received ranging code if Number of received codes > 0 |
| Adjustment parameters indication (API) | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | Shall be included for each received ranging code when Ranging status is 0b01 |
| Timing offset adjustment | 10 bits | Shall be included when Time offset adjustment indication bit is 1 |
| Power level adjustment | 3 bits | Shall be included when Power level adjustment indication bit is 1 |
| Frequency offset adjustment | 8 bits | Shall be included when Frequency offset adjustment indication bit is 1 |

------------------------------------------------------  Text End  ---------------------------------------------------------------

Remedy 2

Modify Table 675 in P802.16m/D2, page 38 as follows:
--------------------------------------------------------- Text Start ---------------------------------------------------------------

15.2.3.2 AAI_RNG-RSP

{Text in between are ignored (no change)}

Table 675 Parameters for AAI_RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Status | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 = success | It shall be included in the AAI_RNG-RSP message |
| STID | The STID, which we call temporary STID, is used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns. |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. |  |
| Timing Adjust Information | Tx timing offset adjustment | If needed |
| Power Adjust Information | Tx power offset adjustment |  |
| Frequency AdjustInformation | Tx frequency offset adjustment |  |

---------------------------------------------------------- Text End ---------------------------------------------------------------

Remedy 3

Modify text in P802.16m/D2, page 178 line 56 as follows:
--------------------------------text start-----------------------------------------

15.2.11.2 Grant

In the bandwidth request procedure, the grant for BW-REQ message is allocated by CDMA Allocation A-MAP IE, which is shown in Table 734. CDMA Allocation A-MAP IE is used for allocation of bandwidth to a user that requested bandwidth using a ranging code or BR code. AMS decodes the IE and checks the MCRC field by its specific 12-bit RA-ID and 4-bit Masking Indicator. The RA-ID is calculated by a hash function with the AMS' random access attributes (frame number index (4 bits), ranging code/BR code index (6 bits) and opportunity index (2 bits) as defined below:

RA-ID = (frame_index | ranging_code_index | opportunity_index)

Masking indicator indicates the identifier used for CRC masking, as shown in Table 735. The allocation shall be in the first UL AAI subframe relevant to the A-MAP region regardless of DL/UL ratio and NAAIsubframe, A-MAP. ABS requests adjustment of timing and power to AMS by using Timing Adjust, Power Level Adjust and Offset Frequency Adjust in CDMA Allocation A-MAP IE for Ranging. The maximum number of the HARQ retransmission is TBD.

Table 734—CDMA Allocation A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { |  | — |
| A-MAP IE Type | 4 | CDMA Allocation A-MAP IE |
| If MCRC is masked with RAID and masking indicator for BR { |  |  |
| Resource assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Power Level Adjust | 4 | Relative change in transmission power level.Signed in units of 1 dB |
| Reserved | TBD |  |
| } |  |  |
| Else if MCRC is masked with RAID and masking indicator for Ranging { |  |  |
| Resource assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation |
| Timing Adjust | 10 | Amount of advance time required to adjust AMS transmission. Unsigned in units of 1/Fs |
| Power Level Adjust | 4 | Relative change in transmission power level.Signed in units of 1 dB |
| Offset Frequency Adjust | 6 | Relative change in transmission frequency.Signed in units of Hz |
| Reserved | TBD |  |
| } |  |  |
| MCRC | 16 | CRC masked by RA-ID and Masking Indicator |
| } |  |  |

--------------------------------text end-----------------------------------------