| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the AAI_RNG-REQ/RSP Message (15.2.3.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30a on the Draft Amendment (IEEE P802.16m/D2) |  |
| Abstract | This contribution proposes modification to the AAI_RNG-REQ/RSP messages |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the AAI_RNG-REQ/RSP Message (15.2.3.1)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Introduction

The 12-bit Random Access ID (RA-ID) used to identify the CDMA Allocation A-MAP IE can also be used to identify the AAI_RNG-RSP message when the STID has not been assigned to the particular AMS. Therefore, MS_Random and temporal STID are not required in AAI_RNG-REQ/RSP messages.

- Proposed Text

Modify Table 674 in P802.16m/D2, page 36 as follows:
--------------------------------------------------------- Text Start ---------------------------------------------------------------

15.2.3.1 AAI_RNG-REQ

{Text in between are ignored (no change)}

Table 674 Parameters for AAI_RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC Version | Version number of IEEE 802.16 supported by the AMS | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| {Remaining fields are ignored (no change) } |  |  |

---------------------------------------------------------- Text End ---------------------------------------------------------------

Modify Table 675 in P802.16m/D2, page 38 as follows:
--------------------------------------------------------- Text Start ---------------------------------------------------------------

15.2.3.2 AAI_RNG-RSP

{Text in between are ignored (no change)}

Table 675 Parameters for AAI_RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Status | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 = success | It shall be included in the AAI_RNG-RSP message |
| RA-IDSTID | The STID, which we call temporary STID, is used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns. The value of the RA-ID is identical to the one received in the corresponding CDMA Allocation A-MAP IE. |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. |  |
| {Remaining fields are ignored (no change) } |  |  |

---------------------------------------------------------- Text End ---------------------------------------------------------------