| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Limitations on burst allocations to an AMS |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Tom Harel Intel corp. | Voice: +972-54-7884132 E-mail: tom.harel@intel.com |
| Re: | Comments on IEEE P802.16m/D2 for IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | The contribution proposes limitation on allocations to an AMS |  |
| Purpose | To be discussed and adopted by the IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Limitations on burst allocations to an AMS

Tom Harel

Intel corp.

# Background

Since the scheduler is part of the ABS, its decisions are naturally adapted to the ABS processing capabilities. For the AMS it is desired to limit the possible allocation scenarios to avoid implementation complexity without substantial gain.

The proposed limitations are shown in the table below, with comparison to WiMAX-1.0 and LTE for reference. The limitations apply to the allocations for one AMS. For AMS that supports Multi-Carrier aggregation, the limitations are applicable for each carrier separately.

|  | Limit on | Proposed value | Rational | WiMAX | LTE |
| --- | --- | --- | --- | --- | --- |
|  | Number DL data bursts in subframe(*) | ≤3 | 1 burst for broadcast transmission, a new burst and burst re-transmission (or otherwise 2 bursts for different QoS) | 5 per burst | 1 |
|  | Out of the DL data bursts in subframe, number of Long TTI bursts | ≤2 | DL Long TTI is useful for subband selection | N/A | 0 |
|  | Number of UL data bursts in subframe(**) | ≤2 | A new burst and a re-transmission (or otherwise Long TTI and default TTI) | 2 per frame | 1 |
|  | Number of A-A-MAP IEs for AMS per subframe(***) | ≤6 | Allocate 2-3 DL bursts, ~2 UL bursts and UL control channels |  | 4 |
|  | Sum of allocation sizes (in LRUs) of bursts that are in subframe (as Default TTI or part of Long TTI) | ≤# of LRUs in subframe | Long TTI isn’t intended to be used for allocating huge bursts, but for exploiting subchannelization gain (UL only) and opportunistic scheduling. | N/A | N/A |
|  | Number of UL Fast Feedback channels (SFBCH and PFBCH) in frame | ≤2 | All MIMO feedback modes can be supported by 1 SFBCH and 1 PFBCH | N/A | N/A |

(*) Includes unicast and broadcast (control and E-MBS), first transmission and HARQ re-transmissions, default TTI and any portion of Long TTI bursts. Doesn’t include control channels (A-MAP).

(**) Includes first transmission and HARQ re-transmissions, default TTI and any portion of Long TTI bursts. Doesn’t include UL control channels.

(***) Includes all of the STIDs that are required to be checked by the AMS (its STID, group ID of GRA and composite PA, Broadcast ID). For a segmentable IE (IE that occupies more than 1 MLRU in QPSK ½), each segment is counted as 1 IE.

The values in the table are the default limitation values that can be overwritten during the capability negotiation.

# References

[1] P802.16m/D2: Advanced Air Interface