| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Capability Negotiation for Location Based Service(15.2.3.7) |  |
| Date Submitted | 2009-11-13 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Chuan Zhong,Fangmin Xu Beijing Samsung Telecom R&D Center Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 |  |
| Abstract | This contribution proposes a method to capability negotiation procedures between AMS and ABS for Location Based Service. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Capability Negotiation for Location Based Service

Lei Zhou, Xufeng Zheng, Chuan Zhong, Fangmin Xu

Beijing Samsung Telecom R&D Center

Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes a method to capability negotiation procedures between AMS and ABS for Location Based Service. Based on system description document (SDD) in 802.16m-08_003r9a, AMS need support A-GPS, so AMS need negotiate A-GPS capability with ABS. For other positioning capability such as U-TDOA method and D-TDOA method, AMS and ABS can support them ,so these capabilities needn’t be negotiated between ABS and AMS.

During network entry, the ABS should negotiate A-GPS method with AMS by AAI_REG-RSP message.

The Proposed Text in AWD

-------------------------------  Text Start  ---------------------------------------------------

[adopt the following text modification started from line# 41 Page 43 in P802.16m/D2]

Table 677—Parameters for AAI_ REG-RSP

| Name |  |  | Value |
| --- | --- | --- | --- |
| MS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | the number of unique SN values; default =2^10 |
|  |  | ARQ_WINDOW_SIZE | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |
|  |  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | the maximum time interval an ARQ block shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEOUT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size |
|  | LBS Capabilities | TBD Capability for support A-GPS method | TBD 0b0: no support 0b1:support |
|  |  | TBD | TBD |
|  | Co-located coexistence capability supported |  | Indicates if co-located coexistence is supported. |
|  | Capabilities for MS measurement |  | TBD |
|  | Capabilities for MIMO support |  | TBD |

----------------------------------------  Text End  -----------------------------------------------