| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Enhanced LBS Support (15.8.3) |  |
| Date Submitted | 2009-11-07 |  |
| Source(s) | Lei Zhou, Fangmin Xu, Xufeng Zheng,Zheng Zhao Beijing Samsung Telecom R&D Center Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 |  |
| Abstract | This contribution proposes solution of E-LBS zone to enhance performance of LBS positioning and control messages to support E-LBS zone |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Capability Negotiation for Location Based Service

Lei Zhou, Fangmin Xu, Xufeng Zheng, Zheng Zhao

Beijing Samsung Telecom R&D Center

Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes solution of E-LBS zone to enhance performance of LBS positioning and control messages to support E-LBS zone Based on 802.16m Draft Amendment, IEEE P802.16m/D2, E-LBS zone need be supported in order to enhance performance of LBS positioning. At the same time, we also consider effect of E-LBS zone on HARQ timing and can’t affect current structure of subframe ( 5,6,7 symbols) and pilot pattern. Structure of distributed E-LBS zone and control messages related on E-LBS in SCD messages are proposed. The detail structure of E-LBS zone is as follows in the proposed text in AWD.

The Proposed Text in AWD

-------------------------------  Text Start  ---------------------------------------------------

-------------------------------  Proposal 1 Start -----------------------------------------------

[adopt the following text modification started from line# 6 Page 581 in P802.16m/D2]

15.8.3 Enhanced LBS Support

The Enhanced LBS (ELBS) is an optional LBS capability involving the following functions:

•ABS ability to transmit special ELBS waveforms to be used for location specific measurements to enable more accurate location determination.

•ABS’s ability to signal to the AMS the location of the special ELBS waveforms.

•AMS’s ability to detect, measure and report relevant measurements to the ABS.

15.8.3.1 Position of D-LBS zone in Frame Structure

The Enhanced LBS (ELBS) is an optional LBS capability. Example of the advanced air interface basic frame structure with E-LBS-zone is illustrated in Figure NNN. The E-LBS zone transmission period (DLZP)  is measured in superframes units. E-LBS zone consists of 4 distributed OFDM symbols and 4 symbols of E-LBS zone are respectively allocated to 4 frame in one superframe. When the D-LBS zone is allocated inside a superframe, its location shall be configurable or predefined and placed in a superframe. Location of every symbol in E-LBS-zone is configurable in first symbol of Type I or Type II subframe or last symbol of Type I or Type II subframe.

Figure NNN Advanced air interface basic frame structure with E-LBS-zone

15.8.3.2 MAC message support for E-LBS-zone

The location of E-LBS-zone within superframe are variable parameters in the system which can be signaled to SCD messages(see15.2.3.28).

-------------------------------  Proposal 1 End -----------------------------------------------

-------------------------------  Proposal 2 Start -----------------------------------------------

[adopt the following text modification started from line# 62 Page77 in P802.16m/D2]

15.2.3.28 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration ChangeINTEGER (0 ~ 15)

BS_Restart_CountINTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINR averagging parameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ONINTEGER (0..1) OPTIONAL

OL-Rank1-ConfigurationINTEGER (0..7) OPTIONAL

SB-OL-Region-2SizeINTEGER (0..15) OPTIONAL

}

E-LBS-ZONE_Parameters::= SEQUENCE {

LBS_zone-ON INTEGER(0..1) OPTIONAL

LBS_subframe_position INTEGER(0..7) OPTIONAL

LBS_symbol_position INTEGER(0..7) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3INTEGER (0..15) OPTIONAL

Alpha (a)INTEGER (0.. 7) OPTIONAL

Beta (ß)INTEGER (0..1) OPTIONAL

SINRminINTERGER (0..15) OPTIONAL

}

-- ASN1STOP

-------------------------------  Proposal 2 End -----------------------------------------------

----------------------------------------  Text End  --------------------------------------------