| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Enhanced LBS Support (15.8.3) |  |
| Date Submitted | 2009-11-13 |  |
| Source(s) | Lei Zhou, Fangmin Xu, Xufeng Zheng,Zheng Zhao Beijing Samsung Telecom R&D Center Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 |  |
| Abstract | This contribution proposes solution of E-LBS zone to enhance performance of LBS positioning and control messages to support E-LBS zone |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Capability Negotiation for Location Based Service

Lei Zhou, Fangmin Xu, Xufeng Zheng, Zheng Zhao

Beijing Samsung Telecom R&D Center

Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes solution of E-LBS zone to enhance performance of LBS positioning and control messages to support E-LBS zone Based on 802.16m Draft Amendment, IEEE P802.16m/D2, E-LBS zone need be supported in order to enhance performance of LBS positioning. At the same time, we also consider effect of E-LBS zone on HARQ timing and can’t affect current structure of subframe ( 5,6,7 symbols) and pilot pattern. Structure of distributed E-LBS zone and control messages related on E-LBS in SCD messages are proposed. The detail structure of E-LBS zone is as follows in the proposed text in AWD.

The detailed simulation parameter is shown in table 1. Simulation results in 19 cells of proposed E-LBS-zone is shown in Figure 1.

| Scenario/Parameters | Baseline |
| --- | --- |
| Cells /Sectors | 19/3 |
| Frequency reuse | 3 |
| Reference signal | SA-preamble/D2 |
| Site to Site Distance | 1.5Km |
| Carrier | 2.5GHz |
| Operating Bandwidth | 10MHz |
| BS & MS Height | 32m&1.5m |
| BS &MS Power | 46dBm&23dBm |
| Pass Loss Model | Loss(dB)=130.19+37.6log10( R ) |
| Lognormal Shadowing Std.Dev | 8dB |
| Channel | PB |
| BS&MS Antenna Gain | Refer to LBS simulation Assumption doc |
| BS&MS Cable Loss | Refer to LBS simulation Assumption doc |
| Noise Figure | Refer to LBS simulation Assumption doc |

Table 1 Simulation parameter

Figure 1 Simulation result

The Proposed Text in AWD

-------------------------------  Text Start  ---------------------------------------------------

-------------------------------  Proposal 1 Start -----------------------------------------------

[adopt the following text modification started from line# 6 Page 581 in P802.16m/D2]

15.8.3 Enhanced LBS Support

The Enhanced LBS (ELBS) is an optional LBS capability involving the following functions:

•ABS ability to transmit special ELBS waveforms to be used for location specific measurements to enable more accurate location determination.

•ABS’s ability to signal to the AMS the location of the special ELBS waveforms.

•AMS’s ability to detect, measure and report relevant measurements to the ABS.

15.8.3.1 Position of D-LBS zone in Frame Structure

The Enhanced LBS (ELBS) is an optional LBS capability. Example of the advanced air interface basic frame structure with E-LBS-zone is illustrated in Figure NNN. The E-LBS zone transmission period (DLZP)  is measured in superframes units. E-LBS zone consists of 4 distributed OFDM symbols and 4 symbols of E-LBS zone are respectively allocated to 4 frame in one superframe. When the D-LBS zone is allocated inside a superframe, its location shall be configurable or predefined and placed in a superframe. Location of every symbol in E-LBS-zone is configurable in first symbol of Type I or Type II subframe or last symbol of Type I or Type II subframe.

Figure NNN-1 Advanced air interface basic frame structure with E-LBS-zone

15.8.3.2 MAC message support for E-LBS-zone

The location of E-LBS-zone within superframe and its transmission mode, periodicity are variable parameters in the system which can be signaled to SCD messages (see15.2.3.28) and AAI_LBS-REQ.

Transmission mode of E-LBS-zone includes continuous mode, periodic mode, one time mode. Detailed transmission mode is illustrated in Figure NNN-2

Figure NNN-2 E-LBS-Zone transmission Mode

E-LBS-Zone related message in AAI_LBS-REQ

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| AAI_LBS-REQ format () { | - | - |
| LBS_method_type | - | 0b1:E-LBS-Zone |
| LBS_frame_position | 2 | 0b00: 1st frame 0b01:2nd frame 0b10: 3rd frame 0b11: 4th frame |
| LBS_subframe_position | 3 | 0b000: 1st subframe 0b001:2nd subframe 0b010: 3rd subframe 0b011: 4th subframe 0b100:5th subframe 0b101:6th subframe |
| LBS_symbol position | 3 | 0b000: 1st symbol 0b001:2nd symbol 0b010: 3rd symbol 0b011: 4th symbol 0b100:5th symbol 0b101:6th symbol |
| LBS-Zone mode | 2 | 0b00: Continuous mode 0b01: Periodic mode 0b10: one-time mode |
| Start Super frame number | 8 | Determine location of LBS-zone |
| LBS-Zone_duration | 8 | Duration of E-LBS-zone period |
| LBS-Zone_period | 8 | E-LBS-Zone period during the duration |
| } |  |  |

15.8.3.2 Transmission structure of Reference signal in E-LBS-zone

The D-LBS zone transmission plan specifies on which OFDMA symbol of D-LBS zone the SA-Preambles shall be transmitted. To define transmission plan the existing set of SA-Preambles shall be partitioned into 6 reference signals location (RSs). The IDcells that belong to the i-th RS (IDcellRSi) shall be defined by equation (1) and equation (2) below:

IDcellRSi=256⋅n+IdxRSi;		  (1)

where i indicates the i-th reference signal location (RSi), i=0, 1, ..., 6; n is the segment index defined in section 15.3.6.1.2, and IdxRSi is the index belonging to the i-th RSi that spans the following set of values [i : 6 : 255]. To determine the IDcells that belong to i-th RS, the IdxRSi index shall start from i and increments by 6 up to 255 for each of the segment index n =0,1,2.

The Predefined E-LBS zone transmission structure is specified by  and is shown in Figure NN.

Figure NN Predefined E-LBS zone transmission Structure

Table 1. Predefined E-LBS zone transmission Structure

| Segment index n (Allocated carrier sets) | E-LBS zone symbol number |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| Carrier Set n = 0 | RS 0 | RS 3 | RS 1 | RS 4 |
| Carrier Set n = 1 | RS 1 | RS4 | RS 2 | RS 5 |
| Carrier Set n = 2 | RS 2 | RS5 | RS 0 | RS 3 |

An ABS shall transmit the SA-Preamble sequence on the given symbol/carrier set only if its IDcell matches to corresponding RS and carrier set as shown in

-------------------------------  Proposal 1 End -----------------------------------------------

-------------------------------  Proposal 2 Start -----------------------------------------------

[adopt the following text modification started from line# 62 Page77 in P802.16m/D2]

15.2.3.28 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration ChangeINTEGER (0 ~ 15)

BS_Restart_CountINTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINR averagging parameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ONINTEGER (0..1) OPTIONAL

OL-Rank1-ConfigurationINTEGER (0..7) OPTIONAL

SB-OL-Region-2SizeINTEGER (0..15) OPTIONAL

}

E-LBS-ZONE_Parameters::= SEQUENCE {

LBS_zone-ON INTEGER(0..1) OPTIONAL

LBS_subframe_position INTEGER(0..7) OPTIONAL

LBS_symbol_position INTEGER(0..7) OPTIONAL

LBS_zone_start_superframe_number INTEGER(0..255) OPTIONAL

LBS_zone_duration INTEGER(0..255) OPTIONAL

LBS-Zone_period INTEGER(0..255) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3INTEGER (0..15) OPTIONAL

Alpha (a)INTEGER (0.. 7) OPTIONAL

Beta (ß)INTEGER (0..1) OPTIONAL

SINRminINTERGER (0..15) OPTIONAL

}

-- ASN1STOP

-------------------------------  Proposal 2 End -----------------------------------------------

----------------------------------------  Text End  --------------------------------------------