| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Key usage during 16m-16e zone switching (15.2.5.2.4) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Chengyan Feng Yang Liu Mary Chion ZTE Corporation | E-mail: feng.chengyan@zte.com.cn liu.yang8@zte.com.cn mchion@zteusa.com |
| Re: | IEEE 802.16m-09/0057, “IEEE 802.16 Working Group Letter Ballot #30a: Announcement” Target topic: “15.2.5.2.4” |  |
| Abstract | This contribution proposes the texts for AMS privacy section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for IEEE 802.16m D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Key usage during 16m-16e zone switching (15.2.5.2.4)

Chengyan Feng, Yang Liu, Mary Chion

ZTE Corporation

# 1. Introduction

HO procedure requires security update to achieve seamless operation. This contribution proposes the Key usage during zone switch from LZone to MZone.

# 2. Text Proposal

======================== Start of Proposed Text =====================

15.2.5.2.4.6 Key update during zone switching from LZone to MZone

ABS shall include Nonce_ABS in the zone switching information.

After receiving NONCE_ABS, AMS shall generate NONCE_AMS. AMS shall increment CMAC_Key_COUNT and derive PMK based on NONCE_ABS and NONC_AMS. And then AK, CMAC keys and TEKs are derived. AMS shall include NONCE_AMS and NONCE_ABS in the following AAI_RNG-REQ message with CMAC protection.

Upon receipt of AAI_RNG-REQ message, ABS shall verify that the received NONCE_ABS matches the value provided by the ABS in the zone switching information. If the ABS_Random value does not match, the ABS shall ignore the message. If matches, ABS shall derive AMSID*, and increment CMAC_KEY_COUNT and derive PMK, AK, CMAC keys and TEKs based on the new generated AMSID*to be used in MZone during network reentry procedure as described in Section 15.2.5.2.3.1. The ABS shall verify the CMAC in the AAI_RNG_REQ message. If the CMAC is invalid, the ABS shall ignore the message. ABS shall send AAI_RNG-RSP message encrypted by the new TEK carrying NONCE_ABS, NONCE_AMS to AMS.

Upon receipt of PKMv2 AAI_RNG-RSP, AMS shall decrypt this message with the corresponding TEK, and then verify this message. If the verification is failed, the SS shall ignore the message.

The AMS and ABS shall also manage the old security context used to maintain communications in LZone before zone switching to MZone finishes.

============================== End of Proposed Text ===============

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010R2, “IEEE 802.16m Amendment Working Document”