| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the IEEE P802.16m/D2：AMS ID Privacy |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Chengyan Feng Yang Liu Mary Chion ZTE Corporation Youngkyo Baek Samsung | E-mail: feng.chengyan@zte.com.cn liu.yang8@zte.com.cn mchion@zteusa.com youngkyo.baek@samsung.com |
| Re: | IEEE 802.16m-09/0057, “IEEE 802.16 Working Group Letter Ballot #30a: Announcement” Target topic: AMS Privacy |  |
| Abstract | This contribution proposes the texts for AMS privacy section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for IEEE 802.16m D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS ID Privacy

Chengyan Feng, Yang Liu, Mary Chion

ZTE Corporation

Youngkyo Baek

Samsung

# 1. Introduction

In AWD P802.16m_D1, in order to protect AMS ID privacy, the real AMSID is deleted from the AAI_RNG-REQ/RSP messages, so the specific field used to distinguish AAI_RNG-REQ/RSP sent by different AMSs is still a problem. An idea that using a Random in AAI_RNG-REQ/RSP to identify AMS is brought forward. But this adds overheads to networks. In this contribution, we propose a solution how to identify an AMS in AAI_RNG-REQ/RSP messages.

# 2. Solution

We suggest AMSID* is transmitted in AAI_RNG-REQ/RSP messages in initial network entry. The AMSID* is the hash value of the real AMSID (ie., AMS MAC Address). In order to avoid exposure in the air interface, AMSID is hashed before transmission. We can use AMSID* to both identify the AMS in AAI_RNG-REQ/RSP messages and protect AMS privacy.

The hash result AMSID* is derived as follows:

AMSID*=Dot16KDF(AMSID, NONCE_AMS, 48)

-NONCE_AMS is a random number of 48-bit generated by AMS before sending AAI_RNG-REQ message, and transmitted to ABS during the following Key Agreement procedure. If the AMS doesn’t receive a successful AAI_RNG-RSP from the ABS, the AMS should re-generate a NONCE_AMS and re-derive the AMSID*, and then send another AAI_RNG-REQ with it to the ABS.

After the successful initial authentication/authorization procedure, the AMS and ABS derive AK based on the AMSID*. And the real AMSID can be deferred to be sent to the ABS in AAI_REG-REQ message in an encryption manner.

Figure 1 shows the overall network entry procedure to Support AMS Privacy in IEEE 802.16m.

Figure 1 Network Initial Entry Procedure to Support AMS Privacy in IEEE 802.16m

# 3. Text Proposal

Replace whole section of 15.2.5.4.1 start from P.114 with text 1

======================== Start of Proposed Text 1=====================

15.2.5.4.1 AMS Identity privacy

AMS identity privacy support is the process of protecting the identity of AMS so that AMS MAC Address (ie., AMSID) is not revealed via air interface. To protect AMSID a hash value of the real AMSID is defined.

In order to avoid exposure in the air interface, the AMS MAC Address is hashed before transmission. The hash result AMSID* is derived as follows:

AMSID*=Dot16KDF(AMSID|80-bit zero padding, NONCE_AMS, 48)

• NONCE_AMS is a random of 48-bit generated by AMS before sending AAI_RNG-REQ message, and transmitted to ABS during the following Key Agreement procedure. If the AMS doesn’t receive a successful AAI_RNG-RSP from the ABS, the AMS should re-generate a NONCE_AMS and re-derive the AMSID*, and then send another AAI_RNG-REQ with it to the ABS.

15.2.5.4.2 AMS location privacy

AMS location privacy support is the process of protecting the mapping between AMS MAC address and STID so that intruders cannot obtain the mapping information between the MAC address and STID. To protect the mapping between STID and AMS MAC address, a temporary STID is assigned during initial ranging process, and is used until STID is allocated.

The STID is assigned during registration process after successful completion of initial authentication/authorization process, and is encrypted during transmission. The temporary STID is released after STID is securely assigned. The STID is used for all remaining transactions. The detailed procedures are described as follows:

AMS generates a new NONCE_AMS and derive AMSID*, then it sends AAI_RNG-REQ carrying the AMSID* to ABS. When ABS receives the AAI_RNG-REQ, it returns AAI_RNG-RSP containing temporary STID (instead of STID) and the AMSID* which the AMS sent. After being assigned, the temporary STID is used for the subsequent network entry procedures until STID is allocated. The real AMSID is transmitted to ABS in AAI_REG-REQ message in an encryption manner. The STID is assigned after the authentication procedure is successfully completed and the assignment message, i,e, AAI_REG-RSP shall be encrypted.  Once the AMS receives the STID via AAA_REG-RSP, it released the temporary STID. The STID is then used for remaining transactions. Figure 403 shows the overall network entry procedures.

Figure 403—Network Entry Procedure to Support AMS Location Privacy in IEEE 802.16m

============================== End of Proposed Text 1 ===============

Replace whole section of 15.2.5.2.3.1 start at P106 with text 2

======================== Start of Proposed Text 2 =====================

15.2.5.2.3.1 Key agreement

The key agreement procedure takes place immediately following authentication/re-authentication or when PMK update is required without re-authentication.

It includes exchange of parameters between the AMS and ABS including NONCEs which are used to derive the PMK from the MSK which was created during authentication.

All other keys are derived from PMK right after or in other situation that requires it like HO or location update/re-entry from idle.

The key agreement procedure (as shown in Figure 398) includes the following steps:

• AMS and NW complete EAP authentication (Authenticator got “EAP Success” from AAA and sent it to AMS).

• The ABS sends AAI_PKM_RSP (key agreement msg#1) to the AMS, the message includes a plaintext random NONCE_BS.

• The AMS derives all security keys from the PMK, AMSID* and other parameters as defined in 15.2.5.2.1.1 and sends AAI_PKM_REQ (key agreement msg#2) including the NONCE_BS and NONCE_MS to the ABS. The AAI_PKM_REQ is integrity protected (CMAC digest using the derived CMAC keys) but not encrypted.

• The ABS takes the NONCE_MS, calculates the keys and verifies the CMAC it received based on the derived keys, if CMAC is verified then the ABS knows it has the same keys which are bind to the AMSID and ABSID, the keys are also fresh due to the 2 NONCE values in the derivation function.

• The ABS then sends to the AMS AAI_PKM_RSP (key agreement msg#3) that includes the NONCE_AMS, NONCE_ABS, the supported SAIDs (0x1 and 0x2) and CMAC digest to prove the possession of the keys and their freshness.

• The AMS verifies the CMAC and derive the TEKs for the supported SAIDs.

In case of initial network entry, once key agreement is completed successfully, the AMS sends to the ABS AAI_REG-REQ that includes the real AMSID as defined in 15.2.15.6.

Note that supplying the AMSID to the ABS allows, among other used of AMSID, for the NW elements to calculate AMSID* whenever a new AK needs to be derived from PMK (HO for example).

Figure 397—Key agreement procedure

============================== End of Proposed Text 2===============

Replace whole section of 15.2.5.2.1.3 start at Page 104 with text 3

======================== Start of Proposed Text 3=====================

15.2.5.2.1.3 AK Derivation

AK is derived from PMK and it belongs to a pair of AMS and ABS.

The AK derivation is done:

AK = Dot16KDF (PMK, AMSID*|ABSID|CMAC_KEY_COUNT|”AK”, 160)

Where:

•  AMSID* - a permutation of AMSID (i.e. AMS MAC address) sent by AMS to ABS in initial AAI_RNG-REQ message, this is used to bind the key to the AMSID and AMSID* is derived from the formula AMSID* = Dot16KDF(AMSID |80-bit zero padding, NONCE_AMS ,48).

•  CMAC_KEY_COUNT – a counter which is used to ensure different AKs for the same ABS-AMS pairs across handovers, the counter is managed as described in section XXX.After (re)authentication the counter value is set to “0”.

============================== End of Proposed Text 3===============

Replace whole section of 15.2.5.2.4.4 at P.110 with text 4.

======================== Start of Proposed Text 4=====================

15.2.5.2.4.4 Key update during Handover

During Handover (with handover process optimization bitmap bit#1=1 “omit PKM”), AK, CMAC keys and TEKs shall be derived by the target ABS and AMS respectively as described in 15.2.5.2. In particular,

•In AK derivation, the CMAC_KEY_COUNT is managed on AMS and target ABS sides in the same way as in Section 7.2.2.2.6.1 and 7.2.2.2.9.1.

•In TEK derivation, COUNTER_TEK is set to be 0 and 1, in order to generate two new TEKs to be used at the target ABS. Corresponding EKS is also reset to be 0 and 1 respectively.

When Seamless_Handover Flag = 1 in AAI_HO-CMD, the AMS and target ABS may use derived TEKs for the target ABS to resume data communications before network reentry procedure finishes.

When the Network_Reentry_Mode = 1, for which the AMS is to maintain communications with serving ABS during network reentry at the target ABS, the AMS shall manage two set of key context for AK, CMAC keys and TEKs, where the context associated with the serving ABS is used to maintain communications with serving ABS until Disconnection Time, and the “new” context associated with the target ABS is used to perform required network reentry procedures with target ABS. The AMS discards key context associated with the serving ABS when the network reentry procedure finishes. The serving ABS discards this MS’s key context (along with other MAC context associated with the AMS) upon either the expiration of Resource_Retain_Time or a HO-Complete signaling from target ABS via backbone.

============================== End of Proposed Text 4===============

Replace whole section of 15.2.5.2.4.5 at P. with text 5.

======================== Start of Proposed Text 5=====================

15.2.5.2.4.5 Key usage during Location Update and Network re-entry from Idle mode

During Location Update or Network re-entry from idle mode, AK, CMAC keys and TEKs shall be derived by the network and AMS respectively if the AMS and the network share valid security context (e.g. PMK).

In particular,

•In AK derivation, the CMAC_KEY_COUNT is managed on AMS and target ABS sides in the same way as in Section 7.2.2.2.9.1.

•In TEK derivation, COUNTER_TEK is set to be 0 and 1, in order to generate two new TEKs to be used at the preferred target ABS. Corresponding EKS is also reset to be 0 and 1 respectively.

============================== End of Proposed Text 5 ===============

Replace the line 47 to line 58 of 15.2.6.4.1.2.1 with text 6

======================== Start of Proposed Text 6=====================

The Zone Switch TLV shall include the following:

• MZone A-Preamble index.

• Time offset between LZone and MZone

• Action Time: Action time of zone switch from LZone to MZone. AMS performs zone switch at Action Time. If HO_Reentry_Mode=0, ABS stops all resource allocation for the AMS at LZone.

• Zone Switch Mode: If set to 1, the AMS maintains its data communication with the ABS in LZone while performing network reentry in MZone; otherwise it breaks data communication in LZone before performing network reentry in MZone.

• NONCE_ABS: It is used to derive a new PMK to be used in Mzone.

============================== End of Proposed Text 6===============

Revise line 7- 14 at P. 36, table 674 as shown in blue in text 7

======================== Start of Proposed Text  7=====================

Table 674—parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| AMSID* MS_Random | It’s the hash value of AMSID in order to protect AMS privacy, which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time | It shall be included when the AMS is attempting network entry without its STID/DID which the ABS/Paging Controller assigns. It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |
| … | … | … |

======================== end of Proposed Text 7=====================

Revise line 38- 47, P.38, at table 675 as shown in text 8 (blue)

======================== Start of Proposed Text 8=====================

15.2.3.2 AAI_RNG-RSP

Table 675—parameters for AAI_ RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Status | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 = success | It shall be included in the AAI_RNG-RSP message |
| STID | The STID, which we call temporary STID, is used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message when the AMS is not assigned its STID/DID yet. It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message when the AMS is not assigned its STID yet |
| AMSID* MS_Random | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent |  |
| … | … | … |

============================== End of Proposed Text 8===============

Replace 708 with text 9 at P. 87

======================== Start of Proposed Text 9=====================

Table 708—PKMv3 Key_Agreement-MSG#2 message attributes

| Attribute | Contents |
| --- | --- |
| NONCE_ABS | A freshly generated random number of 64 bits contained in PKMv3 Key_Agreement-MSG#1 message. |
| NONCE_AMS | A freshly generated random number of 64 bits |
| Key Sequence Number | New AK sequence number |
| AMSID* | AMSID*=Dot16KDF(AMSID¦80-bit zero padding, NONCE_AMS, 48) |
| Security negotiation parameters | The requesting AMS's security capabilities |
| CMAC digest | Message digest calculated using new AK. |

======================== End of Proposed Text 9=====================

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE P802.16m/D2, “Advanced Air Interface(draft 2)”