| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL HARQ Channel Indication in IEEE 802.16m/D2 (15.2.14) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Ji-Yun Seol Samsung Electronics | jiyun.seol@samsung.com |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot#30a” Target topic: “IEEE P802.16m/D2 section 15.2.14 ”. |  |
| Abstract | The contribution proposes to extend the size of ACID in the UL A-MAP IE to the full bits to cover up to the maximum number of UL HARQ channels, which is 16 in the current draft standard 802.16m/D2. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL HARQ Channel Indication in IEEE 802.16m/D2 (15.2.14)

Ji-Yun Seol

Samsung Electronics

- Introduction

This contribution proposes a change to the UL HARQ channel indication method in the current 802.16m draft standard [1]. The ‘ACID’ field in the A-MAP IE is used as HARQ channel identifier assigned to the allocated burst. The maximum number of HARQ channels per AMS is 16 for both DL and UL HARQ. For DL HARQ, ACID of 4bits is used to discriminate among 16 HARQ channels, and there follows explicit one-to-one mapping between ACID and HARQ channel (and also the HARQ buffer used for the specific HARQ channel) as a result. On the other hand, since only 3bits are used for ACID in the uplink, ‘Subframe_index’ should be taken into account as well to make distinction among 16 HARQ channels. That is, there could be UL bursts allocated with the same ACID (= 0~7) in the UL A-MAP IE but in different subframes, which are mapped to different HARQ channels respectively. However, this overhead reduction of UL ACID by 1bit may cause ambiguity in certain situations and increase complexity to handle the case accordingly:

- A UL burst with ACID ‘N’ in subframe ‘K’ and one with ACID ‘N’ in subframe ‘K+1’ are considered as different ones associated with different HARQ channels respectively. Thus, there could be 8 ACIDs per subframe, and ABS shall allocate UL HARQ bursts with different ACIDs and/or subframe indices within the limitation of the maximum number of UL HARQ channels, which is 16.

- When status mismatch of a HARQ channel between ABS and AMS takes place (e.g., due to loss of UL A-MAP IE or false detection of ACK_RSP), ambiguity/complexity in the HARQ buffer management might be caused because of inexplicit mapping between ACID and HARQ channel (and HARQ buffer associated with the HARQ channel). For example, when an AMS performs UL HARQ retransmission for a certain UL HARQ channel (indicated with ACID and Subframe_index) implicitly/synchronously up to N_MAX_ReTx, but ABS has taken it already as a successfully decoded one and has released the HARQ channel from the specific subframe but allocates a new HARQ burst with the same ACID in a different subframe, AMS would consider there being two different HARQ channels active.

- When ABS schedules UL HARQ burst(s) dynamically over different subframes, if ABS intends to stop UL HARQ retransmission process for a certain UL HARQ channel (indicated with ACID in a specific subframe) before the maximum retransmission ‘N_MAX_ReTx’ is reached and to allocate a new UL HARQ burst (not clear if this is allowed in the current AWD though), the ABS has to transmit UL A-MAP IE with the same ACID and AI_SN toggled only in the same subframe, which puts some restriction in ABS scheduling.

- When it comes to long TTI HARQ burst spanning over several UL subframes, it is ambiguous which subframe it is associated with. If only long TTI HARQ bursts are used, the number of UL HARQ channels that could be allocated will be limited by the bit size of ACID (that is, only 8 HARQ channels could be allocated)

- Further consideration of mapping between HARQ channel and connection might be needed to reserve certain HARQ channel(s) to be used only for specific UL connection. (e.g., HARQ channels used for PA/GRA in association with specific connection such as VoIP) In this case, there should be explicit mapping between ACID and HARQ channel as well as between HARQ channel and UL connection. However, lack of explicit mapping between UL ACID and HARQ channel may cause problem when the connection-to-HARQ channel mapping is considered.

Moreover, there are discrepancy/errors regarding UL HARQ channel discrimination/indication in the standard.

- Currently, UL HARQ channel is discriminated/indicated by ACID of 3bits and subframe index

- In p.389, l.5, Table 816: ACID field in UL Composite PA-MAP IE is 4bits

- In p.243, l.51, the CLC operation stated in the last paragraph of the page and described in Figure 451 cannot be supported when using ACID of 3bits (due to the issues addressed above). That is, if the retransmission colliding with the CLC active interval is rescheduled to a different subframe, MS cannot tell if it is the rescheduled allocation for the colliding retransmission since the ACIDs among different subframes are regarded as different HARQ channel according to the current draft standard.

In this context, it is proposed to extend the size of ACID field in the UL A-MAP IE to full bits (= 4bits) to cover the maximum number of UL HARQ channels, which is 16 in the current draft D2 standard.

- References

[1] IEEE P802.16m/D2, October 2009

- Proposed Text Changes

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

[Change 1: Modify the text in section 15.2.14, line 36~47, page 197 as follows]

- - - - - - - - - - - - - - - - - - - - - - - - - - - Begin Proposed Text  - - - - - - - - - - - - - - - - - - - - - - - -

15.2.14 HARQ Functions

HARQ shall be used for unicast data traffic in both downlink and uplink. The HARQ shall be based on a stop-and-wait protocol. ABS and AMS shall be capable of maintaining multiple HARQ channels. DL HARQ channels are identified by downlink HARQ channel identifier (ACID). UL HARQ channels are identified by uplink HARQ channel identifier (ACID) both ACID and the index of UL subframe in which UL HARQ data burst is transmitted. Multiple UL HARQ channels in the same UL subframe shall be identified by different ACIDs, and UL HARQ channels in different UL subframes shall be identified by the index of UL subframe when the same ACID is addressed to them.

- - - - - - - - - - - - - - - - - - - - - - - - - - - End Proposed Text - - - - - - - - - - - - - - - - - - - - - - - - -

[Change 2: Change the size of ACID field in Table 802 UL basic assignment A-MAP IE, line 54, page 365 as follows]

- - - - - - - - - - - - - - - - - - - - - - - - - - - Begin Proposed Text  - - - - - - - - - - - - - - - - - - - - - - - -

Table 802—UL basic assignment A-MAP IE

| … | … | … |
| --- | --- | --- |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocatedresource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframesfor TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | [4] | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 34 | HARQ channel identifier |
| Reserved | TBD | Reserved bits |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

- - - - - - - - - - - - - - - - - - - - - - - - - - - End Proposed Text - - - - - - - - - - - - - - - - - - - - - - - - -

[Change 3: Change the size of ACID field in Table 815 UL Individual Persistent A-MAP IE, line 48, page 387 as follows]

- - - - - - - - - - - - - - - - - - - - - - - - - - - Begin Proposed Text  - - - - - - - - - - - - - - - - - - - - - - - -

Table 815—UL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| … | … | … |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation |
| ACID | 34 | HARQ channel identifier |
| } |  |  |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

- - - - - - - - - - - - - - - - - - - - - - - - - - - End Proposed Text - - - - - - - - - - - - - - - - - - - - - - - - -