| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on S-SFH SP IE update in sleep mode (15.2.16.2.3.1.x) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak LG Electronics | gwpark@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | Proposed text changes for system configuration information acquisition in sleep mode |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on S-SFH SP IE update in sleep mode

Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics

- Introduction

This contribution proposes the text describing the sleep mode operation when a 16m ABS updates the system configuration information.

- Modifications and key-descriptions

- Clarification of the system configuration information acquisition

  - AAI_TRF-IND Request message

    - In case ABS updates the S-SFH SP IE, if the AMS does not receive the AAI_TRF-IND message during the listening window, after updating the S-SFH SP IE, AMS shall transmit the AAI_TRF-IND request message.

    - Problem: All the AMS which did not receive the AAI_TRF-IND message shall transmit the AAI_TRF-IND request message. This overhead is large.

Figure 1. Transmission of the AAI_TRF-IND request

  - Motivation: It is necessary for reducing the signaling overhead of AAI_TRF_IND-REQ/RSP message.

- Proposal

  - Transmission of updated system configuration information with AAI_TRF-IND message.

  - Figure 2 shows an example of proposal.

    - Operation of AMS A

      - In case AMS does not update system configuration information and does not receive the AAI_TRF_IND message, it doubles the sleep cycle in previous sleep cycle. During the sleep mode, when the ABS updates the system configuration information, it transmits the updated system configuration information with AAI_TRF-IND message to the sleep mode AMSs. If the maximum transmissions of AAI_TRF-IND message (Transmission of AAI_TRF-IND with updated system configuration information) are exhausted, ABS only transmits the updated system configuration information. If the AMS detects the positive traffic indication, it transmits the AAI_TRF_IND-REQ message to ask an ABS about previous AAI_TRF-IND message which was not received because AMS did not update the system configuration information. If the AMS receives the AAI_TRF_IND-RSP (Positive indication), it reset sleep cycle to initial sleep cycle at the Frame_Number which is indicated in AAI_TRF_IND-RSP message. If the AMS receives the AAI_TRF_IND-RSP (Negative indication), it receives the AAI_TRF-IND message at next listening window.

    - Operation of AMS B

      - In case AMS does not update system configuration information and does not receive the AAI_TRF_IND message, it doubles the sleep cycle in previous sleep cycle. During the sleep mode, when the ABS updates the system configuration information, it transmits the updated system configuration information with AAI_TRF-IND message to the sleep mode AMSs. If the maximum transmissions of AAI_TRF-IND message (Transmission of AAI_TRF-IND with updated system configuration information) are exhausted, ABS only transmits the updated system configuration information. If the AMS detects the negative traffic indication, it does not transmit the AAI_TRF_IND-REQ message to ask an ABS about previous AAI_TRF-IND message which was not received because AMS did not update the system configuration information. Because AMS already doubled sleep cycle in previous sleep cycle due to it does not receive the AAI_TRF-IND message. Therefore, signaling of AAI_TRF_IND-REQ/RSP message can be reduced as number of AMSs which is receiving the negative traffic indication.

    - Operation of AMS C

      - In case the length of AMS’s sleep window is longer than the period of AAI_TRF-IND message transmission with system configuration information, AMS can not receive the AAI_TRF-IND message with updated system configuration. Thus, after the AMS updates the system configuration information and then it transmits the AAI_TRF_IND request to ask  an ABS about previous AAI_TRF-IND message which was not received because AMS did not update the system configuration information.

Figure 2. Update of system configuration information with AAI_TRF-IND message

- Proposed Text

[Accept the following modifications marked in blue.]

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add proposed text to line 55, page 226]

15.2.16.2.3.1.x Traffic Indication during the updated system configuration information transaction

In case AMS does not update system configuration information and does not receive the AAI_TRF_IND message, it doubles the sleep cycle in previous sleep cycle. During the sleep mode, when the ABS updates the system configuration information, it transmits the AAI_TRF-IND message right after SFH and A-MAP of the 1st AAI subframe whenever ABS transmits the updated system configuration information during the maximum transmissions of the AAI_TRF-IND message. Maximum transmission number of AAI_TRF-IND message is TBD. If the maximum transmissions of the AAI_TRF-IND message are exhausted, ABS only transmits the updated system configuration information. If the AMS receives AAI_TRF-IND message with positive traffic indication, it transmits the AAI_TRF_IND-REQ message to ask an ABS about previous AAI_TRF-IND message which was not received because AMS did not update the system configuration information. If the AMS receives AAI_TRF-IND message with negative traffic indication, it receives the AAI_TRF-IND message at next listening window. If the AMS receives the AAI_TRF_IND-RSP with positive traffic indication as a response to the reception of the AAI_TRF_IND-REQ message, it reset sleep cycle to initial sleep cycle at the Frame_Number which is indicated in AAI_TRF_IND-RSP message. If the AMS receives the AAI_TRF_IND-RSP with negative traffic indication, it receives the AAI_TRF-IND message at next listening window.

----------------------------------------------------------End of the Text-----------------------------------------------------------

- References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”, October 2009