| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Paging Operation in Multi-carrier Operation (15.2.8.2.10.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Giwon Park, Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim and Jin Sam Kwak LG Electronics | E-mail: gwpark@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | This contribution proposes amendment text describing paging operation in multi-carrier system for ABS Type in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Paging Operation in Multi-carrier Operation (15.2.8.2.10.2)

Giwon Park, Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim and Jin Sam Kwak

- LG Electronics

- Introduction

According to D2, each ABS may support multiple carriers. For paging operation in idle mode, when the ABS transmits the paging message, transmitting the paging message in one of its carriers is recommended rather that transmitting the paging message on each carrier to reduce the overhead of duplicated transmission. This contribution proposes the text describing the paging operation when a 16m ABS supports multiple carriers.

- References

- [1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”, October 2009

- Text proposal for inclusion in the P802.16m/D1

========================== Start of Proposed Text ==============================

[Note to Editor: Add the following text after line 33 in Page 159 in 15.2.8.2.10.2]

- 15.2.8.2.10.2 Idle Mode

- 15.2.8.2.10.2.x Paging operation

When an ABS implements multiple carriers it transmits the paging message for an idle mode AMS in one of its carriers. To determine the carrier index, the ABS broadcasts the paging carrier parameters in the AAI_NBR-ADV message. An idle mode AMS determines the carrier index for monitoring the paging message within AMS’s paging listening interval where its paging message is transmitted using the following equations:

Paging carrier index = AMS Temporary ID modulo N

The carriers are indexed in the increasing order of their center frequency, i.e., the carrier with lowest center frequency has the smallest index and the carrier with highest center frequency has the largest index. AMS implicitly knows the value of N based on paging carrier indicator flag in the AAI_NBR-ADV message. The value of N is the number of carriers per PGID in a BS that are used for transmitting paging message for an idle mode MS. This can be all the carriers or a subset of carriers used by a multi-carrier BS.

[Note to Editor: Add the following text after line 30 in Page 49 in 15.2.3.11]

- 15.2.3.11 AAI_NBR-ADV

For each BS, physical carrier configuration parameters are carried as follow

- Number of neighbor BSIDs

- Number of physical carriers

- Physical carrier index

- Paging carrier indicator

- Number of PGIDs

- PGID

Example of AAI_NBR-ADV message including the physical carrier configuration parameters is shown in Table 1.

Table 1: AAI_NBR-ADV Information transmitted by the network to enable procedures for paging in multicarrier operation.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Num Neighbor BSIDs |  |  |
| for (i=0; i< Num Neighbor BSIDs; i++) { |  |  |
| Num_physical carriers |  | Number of physical carriers |
| for (j=0; j< Num_physical carriers; j++) { |  |  |
| Physcial Carrier Index |  |  |
| Paging Carrier Indicator |  | 0: This carrier does not transmit paging message for idle mode MS. 1: This carrier transmits paging message for idle mode MSs. |
| Num_PGIDs |  |  |
| for (k=0 ; k< Num_PGIDs; k++) { |  |  |
| PGID |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

========================== End of Proposed Text ==============================