| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for E-MBS Zone Specific Pilot Patterns for MBSFN |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Ting Liang, Bo Sun, Dengkui Zhu, Kaiying Lv, Ying Liu, Zirong Li, Dihao Ning ZTE Corporation | Voice: Email: {liang.ting12, sun.bo1}@zte.com.cn |
| Re: | IEEE P802.16m/D2: EMBS |  |
| Abstract | The contribution proposes the text of EMBS pilot pattern to be included in amendment document. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for E-MBS Zone Specific Pilot Pattern for MBSFN

# Ting Liang, Bo Sun, Dengkui Zhu, Kaiying Lv, Ying Liu

Zirong Li, Dihao Ning

# ZTE Corporation

# Introduction

In current 802.16 D2 descriptions, E-MBS zone specific pilot is transmitted for MBSFN transmissions. The pilot structures within an E-MBS zone should be supported up to two streams. In SFN transmission, the Delay spread is higher than typical unicast scenarios, and the unicast pilot patterns may be not sufficient. So a new pilot pattern designs is needed for E-MBS zone.

This contribution proposes one pilot pattern for E-MBS zone.

# The structure of MIMO measurement pilot

A physical structure of E-MBS pilot which supported up to two streams for MBSFN transmission is proposed.

Figure 1 Proposed E-MBS pilot patterns in a RU for 2 Stream

Figure1 shows the proposed E-MBS pilot pattern for up to 2 streams which is to be used for E-MBS zone. The blue squares stand for E-MBS pilot for stream1 and the red squares stand for E-MBS pilot for stream2.

# References

[1] IEEE 802.16m-D2, “The Draft IEEE 802.16m System Advanced Air Interface”

[2] “Proposed text of E-MBS Zone Specific Pilot Patterns for MBSFN”, IEEE C802.16m-09/2156r1

# Text proposal for inclusion in the 802.16m D2

-------------------------------  Proposed Text Start  -------------------------------------------------------------------

15.3.5.4.3 E-MBS Zone Specific pilot Patterns

E-MBS zone specific pilots are transmitted for multi-cell multicast broadcast single frequency network (MBSFN) transmissions. An E-MBS zone is a group of ABSs involved in an SFN transmission. The E-MBS zone specific pilots are common inside one E-MBS zone. Synchronous transmission of the same pilots from multiple ABSs helps an AMS to estimate the composite channel response within an E-MBS zone.

The single E-MBS pilot pattern is robust for various cell size MBSFN deployments. The pilot supports channel estimation of up to two transmit streams within an E-MBS zone. The structures of the E-MBS zone specific pilot pattern are shown in FFS figure xxx.

Figure xxx E-MBS pilot patterns in a RU for 2 Stream

-------------------------------  Proposed Text End  -----------------------------------------------------------