| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for IEEE P802.16m/D2: Tproc definition for HARQ Function |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Ning Ding, Guan Yanfeng, Sun Bo ZTE Corporation | Voice: E-mail: {ning.ding, sun.bo1}@zte.com.cn |
| Re | IEEE P802.16m/D2: HARQ Functions Section 15.2.14.2.2 |  |
| Abstract | Defining data burst processing time for A-MAP relevance and HARQ timing in IEEE P80216m/D2 |  |
| Purpose | For discussion and adoption by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for IEEE P802.16m/D2: Tproc Definition for HARQ Function

Ning Ding, Guan Yanfeng, Sun Bo

ZTE Corporation

- Introduction

Regarding A-MAP relevance and HARQ timing in HARQ Functions section, how to inform the data burst processing time (Tproc) between AMS and ABS is not defined, neither the exact value of Tproc. The absence of a certain Tproc will make the A-MAP relevance and HARQ Timing unclear. This will lead to difficulty and complexity to further system implementation.

In an implemented system, both ABS and AMS must know the Tproc of each other to calculate the HARQ timing schedule. If the Tproc could be a variable for different ABS or AMS in the system, a negotiation process needs to be designed for ABS and AMS to inform the TprocBS and TprocMS to each other. While a fixed TprocBS and TprocMS value for both ABS and AMS is relatively simple and practical.

- Solution Advices

To define the data burst processing time (TprocBS and TprocMS), several principles need to be followed:

- Accommodating with the hardware processing speed of AMS and ABS in real world.

- Minimizing the A-MAP relevance delay. When z, v or w equal to 1, the HARQ feedback or retransmission delay is quite long. So these cases should be avoided if possible.

- Deploying a unique Tproc value for TDD and FDD to simplify the implementation.

In SDD, a typical value for MS and BS processing delay is suggested as 1.23ms. And from practical perspective, it’s reasonable to request the processing ability of both AMS and ABS to be less than 1.23ms plus a delta, e.g. one subframe duration.

In a practical system, the Tproc for BS (TprocBS)and Tproc for MS (TprocMS)are typically calculated in term of subframe duration. While in systems with different frame structure, e.g. F=6, or 7, or 8, the time duration of a subframe is variable. It’s reasonable to define different TprocBS and TprocMS value with different frame structure, i.e. F.

We propose the value of the processing time (TprocBS and TprocMS) to be 5m×3/8 = 1.875ms when F=8, 5m×2/7 = 1.43ms when F=7 and 5m×2/6 = 1.667ms when F=6. When transformed into subframe duration format, it is Tproc=3 when F=8, Tproc=2 when F=7, Tproc=2 when F=6. This scheme can make sure the delay to be close and bigger than 1.23ms.  The guard time for some cases, e.g. type-3 subframes, is also considered in above design. The suggested value of TprocBS and TprocMS are suitable for almost all cases.

Figure 1 below shows the difference between the TprocBS= 3 and 4 when F = 8, FDD mode. When TprocBS = 4 and N_TTI=1, and z =1, the AMS has to wait more than 1 frame before it can sent HARQ Feedback. The UL basic assignment IE of Frame i+1, Subframe 5 is sent in Frame i, Subframe 1. However, When Tproc = 3 and N_TTI=1, the AMS just needs to wait 4 subframes. The UL basic assignment IE of Frame i+1, Subframe 5 is sent in Frame i+1, Subframe 1. The distance between UL basic assignment IE and affected subframe becomes much shorter if TprocBS = 3.

Figure 1

With our suggested TprocBS and TprocMS value above, the system performance will be improved by considering time delay and A-MAP relevance. For F= 8, Tproc = 3, the processing time is 5ms*3/8 = 1.875ms. For F= 7, Tproc = 2, the processing time is 5ms*2/7 = 1.43ms. For F= 6, Tproc = 2, the processing time is 5ms*2/6 = 1.6667ms. So AMS just need to attain the requirement of 1.6667ms, then it can fit all ABS settings.

In TDD mode, the TprocBS and TprocMS uses the same value as in FDD mode and these can simplifier the system configuration.

- Reference

[1] IEEE C802.16m-09/0859, “Proposed Text of HARQ Protocol for the 802.16m Amendment”

[2] IEEE C802.16m-09/1130, “Proposed A-MAP Relevance and HARQ Timing for the IEEE 802.16m Amendment (Proposed Text for AWD)”

[3] IEEE C802.16m-P80216m_D2, “Advanced Air Interface (Draft 2)”

[4] IEEE C80216m-09/0034r2, “IEEE 802.16m System Description Document (SDD)”

- 4. Text proposal

[Insert below text in blue in line 13 on the page199, in 15.2.14.2.2, as follows:]

Each transmission time is represented by frame index and subframe index. The frame index shall range from 0 to 3. In FDD, the index of DL or UL subframe shall range from 0 to F-1, where F is the number of subframes per frame. In TDD, the index of DL subframe shall range from 0 to D-1, where D is the number of DL subframes per frame, and the index of UL subframe shall range from 0 to U-1, where U is the number of UL subframes per frame. TprocBS refers to the date burst processing time of ABS and TprocMS refers to the date burst processing time of AMS. The value of TprocBS and TprocMS depend on the value of F. TprocBS = 3 when F = 8, TprocBS = 2 when F = 6 or 7. TprocMS = 3 when F = 8, TprocMS = 2 when F = 6 or 7.