| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications on DL\UL Composite Persistent A-MAP IE |  |
| Date Submitted | 2009-11-4 |  |
| Source(s) | Zhaohua Lu，Bo Sun，Feng Xie，Kun Liu，Lin Chen ZTE Corporation | lu.zhaohua@zte.com.cn sun.bo1@zte.com.cn |
| Re: | Call for LB 30 |  |
| Abstract | The contribution proposed the text relevant to Interference Mitigation section included in the 802.16m amendment |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications on DL Composite Persistent A-MAP IE

Zhaohua Lu, Bo Sun, Feng Xie, Kun Liu, Lin Chen

ZTE Corporation

# Introduction

In current AWD text (P80216m_D2), the DL\UL Composite Persistent A-MAP IE includes redundant field, such as "RCID type", which is a concept from 802.16e and has no meaning in 802.16m. At the same time, it’s enough to assign 12 bits to the field ‘RCID’, which describe the STID of an AMS, and the information of FID need not carried in this field, because this information will limit only one flow in an AMS can use assigned PA resource.

# Proposed text 1

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.3.6.5.2.8 DL PA A-MAP IE

DL Composite PA A-MAP IE

The DL composite persistent A-MAP IE is specified in Table 813.

Table 813 DL Composite Persistent A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID 16-bit composite ID with 12-bit station ID in MSB and 4-bit FID in LSB in RCID format, type defined by RCID Type |
| STID | 12 | Specifies 12 bit station ID of an AMS |
| ... |  |  |

--------------------------------------------------------End of the Text-------------------------------------------------------

# Proposed text2

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.3.6.5.2.9 UL PA A-MAP IE

UL Composite PA A-MAP IE

The UL composite persistent A-MAP IE is specified in Table 816.

Table 816 UL Composite Persistent A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID 16-bit composite ID with 12-bit station ID in MSB and 4-bit FID in LSB in RCID format, type defined by RCID Type |
| STID | 12 | Specifies 12 bit station ID of an AMS |
| ... |  |  |

--------------------------------------------------------End of the Text-------------------------------------------------------