| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed E-MBS Zone specific pilot patterns (15.3.5.4.3) |  |
| Date Submitted | 2009-11-09 |  |
| Source(s) | Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting, Yan-Xiu Zheng ITRI David W. Lin and Ching-Yao Huang NCTU | Jefferywu@itri.org.tw |
| Re: | P802.16m/D2 |  |
| Abstract | In this contribution, we propose the single pilot pattern for MBSFN transmission which is robust to different kinds of channel conditions. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed E-MBS Zone specific pilot patterns

Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting, Yan-Xiu Zheng

ITRI

Hsin-Piao Lin

NTUST

- Introduction

According to IEEE 802.16m/D2 document [1], the E-MBS zone specific pilot shall be transmitted for multicell multicast broadcast single frequency network (MBSFN). The single E-MBS pilot shall provide adequate channel estimation capability and throughput for MBSFN with various inter-site distances (ISD). In this contribution, we propose the single pilot pattern for MBSFN transmission which is robust to different kinds of channel conditions.

- Simulation Results

Following we provide a block error rate (BLER) and throughput comparisons between E-MBS zone specific pilot patterns that are proposed before. Table.1 outlines our simulation parameter setting. The MBSFN channel delay profile is based on evaluation methodology document (EMD) [2]. For G=1/8 cases, MBSFN channel with ISD=5km will induce serious inter-symbol interference (ISI) if there is no ISI mitigation techniques. Therefore, we only conduct Rate-1 SFBC MIMO scheme for evaluation.

Table 1. Simulation parameter setting

Figure 2 shows pilot structures within an 18x6 resource block which are proposed by many companies. There are 16 pilot symbols per RU for Samsung, ZTEA, ZTEB; 18 pilots per RU for ITRI, LGA; 20 pilots per RU for Panasonic and LGB. Since E-MBS only supports up to two stream transmission, all pilot patterns are designed based on this assumption. In our simulation setting, we assume 4 contiguous PRUs are used for channel estimation.

Fig. 2 Compared E-MBS pilot patterns

Following simulation results show the BLER and throughput performance for different pilot patterns. Fig. 3A-3C compares the performance of SFBC transmission in the MBSFN channel with ISD=1.5km and 3 km/hr vehicular speed is assumed. In Fig. 4A-4C, the performance of using SM transmission in the MBSFN channel with ISD=1.5km is shown where 120 km/hr vehicular speed is assumed. Fig. 5A-5C shows the performance by using SFBC in the MBSFN channel with ISD=5km and the vehicular speed is 120km/hr.

For MBSFN ISD=1.5km cases, all pilot patterns perform very similar where the gap is within 0.5 dB. In the case of MBSFN ISD=5km by using low MCS transmission such as QPSK1/2, all pilot pattern still show similar performance where the performance gap is not larger than 1 dB. However, when higher MCS such as 16QAM3/4 or 64QAM1/2 is adopted, the performance of all pilot patterns is deteriorated and the gap in terms of throughput is quite large. We can see ITRI and LG pilot patterns show more robustness in such a stringent condition. Compare all the simulation cases, we suggest to use ITRI’s pattern as E-MBS zone specific pilot.

Fig. 3-A

Fig. 3-B

Fig. 3-C

Fig. 4-A

Fig. 4-B

Fig. 4-C

Fig. 5-A

Fig. 5-B

Fig. 5-C

- Reference

[1] IEEE 802.16m/D2, “IEEE 802.16m Draft Document.”

[2] IEEE 802.16m-07/004r5, “IEEE 802.16m Evaluation Methodology Document.”

- Proposed Text

---------------------------------------------- Start of Proposed Text ------------------------------------

<Please change the text and figure in line 44 on page 312 in the section 15.3.5.4.3>

The single E-MBS pilot pattern is robust for various cell size MBSFN deployments. The pilot supports channel estimation of up to two transmit streams within an E-MBS zone. The structures of the E-MBS zone specific pilot pattern are shown in FFS Figure XXX.

Fig. XXX E-MBS Zone specific pilot patterns for 1 and 2 stream transmission

---------------------------------------------- End of Proposed Text ------------------------------------