| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed E-MBS Zone specific pilot patterns (15.3.5.4.3) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting, Yan-Xiu Zheng ITRI Bo Sun, Liang Ting ZTE Corporation Hsin-Piao Lin NTUST Pei-Kai Liao, Chih-Yuan Lin, Paul Cheng MediaTek Inc. Jinsoo Choi, Dongcheol Kim, HanGyu Cho LG Electronics Isamu Yoshii Panasonic Corporation | Jefferywu@itri.org.tw sun.bo1@zte.com.cn {sun.bo1, liang.ting12}@zte.com.cn |
| Re: | P802.16m/D2 |  |
| Abstract | In this contribution, we propose the single pilot pattern for MBSFN transmission which is robust to different kinds of channel conditions. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed E-MBS Zone specific pilot patterns

Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting, Yan-Xiu Zheng

ITRI

Bo Sun

ZTE Coporation

Hsin-Piao Lin

NTUST

- Introduction

According to IEEE 802.16m/D2 document [1], the E-MBS zone specific pilot shall be transmitted for multicell multicast broadcast single frequency network (MBSFN). The single E-MBS pilot shall provide adequate channel estimation capability and throughput for MBSFN with various inter-site distances (ISD). In this contribution, we propose the single pilot pattern for MBSFN transmission which is robust to different kinds of channel conditions.

- Reference

[1] IEEE 802.16m/D2, “IEEE 802.16m Draft Document.”

[2] IEEE 802.16m-07/004r5, “IEEE 802.16m Evaluation Methodology Document.”

- Proposed Text

---------------------------------------------- Start of Proposed Text ------------------------------------

<Please change the text and figure in line 44 on page 312 in the section 15.3.5.4.3>

The single E-MBS pilot pattern is robust for various cell size MBSFN deployments. The pilot supports channel estimation of up to two transmit streams within an E-MBS zone. The structures of the E-MBS zone specific pilot pattern for type-1 subframes are shown in  FFS. Figure XXX.

The E-MBS zone specific pilot pattern for type-2 subframes is derived from the E-MBS zone specific pilot pattern for type-1 sunframes by duplicating the second symbol and appending to the end of the last symbol.

The E-MBS zone specific pilot pattern for type-3 subframes is derived from the E-MBS zone specific pilot pattern for type-1 sunframes by removing the fourth symbol.

Fig. XXX E-MBS Zone specific pilot patterns for 1 and 2 stream transmission in Type-1 Subframe

---------------------------------------------- End of Proposed Text ------------------------------------