| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on SFH Resource Structure in the IEEE 802.16m Draft. |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Yanfeng Guan, Xianming Chen, Huiying Fang | E-mail: guan.yanfeng@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.5 (DL-PHY) Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-PHY” “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | This contribution makes a clarification on the SFH Resource Structure. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on SFH Resource Structure in the IEEE 802.16m Draft

Yanfeng Guan, Xianming Chen, Huiying Fang

ZTE Corporation

# 1 Introduction

In Draft 2, the subframe including SFH only has one frequency partition, and all PRUs in this subframe will be permuted to the distributed LRUs. However, this issue still needs some clarifications. For example,

1) How many steps are necessary for SFH Resource mapping, and whether the miniband permutation is needed?

2) The PHY structure for resource allocation of the SFH is described in 15.3.5, but nothing is there.

3) Some listed signaling probably means that these parameters need to be signaled.

4）

# 2 SFH Resource Structure

The SFH resource mapping is predefined, any signaling is unnecessary. Because there is no subband and CLRU in the subframe where SFH is located, the resource mapping for SFH doesn’t need the normal steps including the subband partitioning, miniband permutation and CRU/DRU any more. The following figure shows the resource mapping process of SFH for 5MHz system.

Figure 1. Resource Mapping Process in the Subbframe including SFH

# 3 References

[1] IEEE 80216m-P802.16m/D2. Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems. Advanced Air Interface (working document), October, 2009.

# 4 Proposals

Proposal Text #1 suggests making clarifications from line 8 to 46 on the page 332

------------------------------- Text Start ---------------------------------------------------

The Superframe Header (SFH) carries essential system parameters and system configuration information. The SFH is located in the first AAI subframe within a superframe. The SFH uses the last 5 OFDM symbols within the first AAI subframe.

The PHY structure for resource allocation of the SFH is described in Error: Reference source not found. The AAI subframe where SFH is located has one frequency partition.

All PRUs in the first AAI subframe of a superframe have 5 OFDM symbols with the 2 stream pilot pattern defined in Error: Reference source not found. The resource mapping process in the SFH AAI subframe is predefined as follow.

The AAI subframe where SFH is located always has one frequency partition FP0. All NPRU PRUs in the AAI subframe where SFH is located and are permuted to generate the distributed LRUs. The permutation and frequency partition of the SFH AAI subframe can be described by DSAC = 0 (all minibands without subband), DFPC = 0 (reuse 1 only), DCASSB0 = 0 (no subband CRU allocated), and DCASMB0 = 0 (no miniband CRU allocated). Definitions of these parameters are given in Error: Reference source not found.

The SHF SFH occupies the first NSFH distributed LRUs in the first AAI subframe of the superframe where NSFH is no more than 24. The remaining distributed LRUs in the first AAI subframe of the superframe are used for other control and data transmission.

The permutation and frequency partition of the SFH AAI subframe can be described by DSAC = 0 (all miniband without subband), DFPC = 0 (reuse 1 only), DCASSB0) = 0 (no subband CRU allocated), and DCASMB0 = 0 (no miniband CRU allocated). Definitions of these parameters are given in Error: Reference source not found.

The SFH is divided into two parts: Primary Superframe Header (P-SFH) and Secondary Superframe Header (S-SFH).

Table 787 inludes includes the parameters and values for resource allocation of the SFH.

| Parameters | Description | Values |
| --- | --- | --- |
| N SFH | The number of distributed LRUs which are occupied by SFH. Note that N SFH = NP-SFH + NS-SFH | TBD (<= 24) N P-SFH + NS-SFH |
| N P-SFH | The number of distributed LRUs which are occupied by P-SFH | Fixed (value is TBD) |
| N S-SFH | The number of distributed LRUs are occupied by S-SFH | Variable according to the type of S-SFH SP |

------------------------------- Text End ---------------------------------------------------