| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distributed control plane for 16m relays (16.2.3.28 and 16.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Kanchei(Ken) Loa, Chun-Yen Hsu, Youn-Tai Lee Yih-Shen Chen, Paul Cheng Frank Ren Muthu Venkatachalam, Xiangying Yang, Shantidev Mohanty, Jerry Sydir Oleg Marinchenco, Vladimir Yanover Phillip Barber, Ronald Mao Shahab Sayeedi | III MediaTek ITRI Intel Corporation. Alvarion Huawei Motorola |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D3 |  |
| Abstract | Distributed control plane for 16m relays |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distributed control plane for 16m relays

Introduction

This contribution is a revision of C802.16m-09/2347r1 with editial modifications only to meet the text in 802.16m/D3.

In Advance WirelessMAN-OFDMA deployment supporting relay, a distributed control model is used. The user plane protocol architecture is illustrated in Figure 1. Upon the ABS receiving the user plane packets from the ASN, the received packets are mapped to a transport flow of AMS. The mapping mechanism is out of scope of this specification. The user plane packets are first packed into a MPDU with corresponding FID in GMH, and then MPDUs are encapsulated in the relay MPDU of ARS and mapped to a transport flow with similar QoS over the relay link.

Figure 1 ARS user plane protocol stack

The control plane protocol architecture is shown in Figure 2. In this case, the ASN control messages are sent between ASN and ABS, and between ABS and ARS. Upon ABS receiving the ASN control message, it read the AMS ID in the message and translate the ASN control message between the two interfaces by means of encapsulate the ASN control message in a AAI_L2_XFER message (leaving the ASN control message unchanged) and send to the target ARS with FID=FFS. This operation corresponds to an ASN control proxy mechanism. The ASN control proxy operation would be transparent for ASN and ARS. That is, as seen from ASN it looks like as if ARS would be connected via ABS, while from the ARS’s perspective it would look like as if ARS would be talking to ASN directly. Over the relay link, ARS’s ASN control messages are carried in AAI_L2_XFER messages.

Fig. 2 ARS control plane protocol stack

Part-I: Changes to AAI_L2-XFER to support distributed control

[Note to Editor: Modify the following text in 16.2.3.28 in 802.16m/D3 for AAI-L2-xfer message]

16.2.3.28 L2 Transfer message (AAI_L2_XFER)

IEEE 802.16m provides a generic MAC management message called AAI_L2_XFER. This acts as a generic service carrier for various services including, but not limited to:

Device provisioning bootstrap message to AMS, GPS assistance delivery to AMS, ABS(es) geo-location unicast delivery to AMS, 802.21 MIH transfer, messaging service, etc.

This container is also used for 16m messages that are not processed by the ABS or ARS, but are rather processed by network entities beyond the ABS. AAI_L2_XFER shall be transmitted only by authenticated AMSs.

The format of AAI_L2_XFER message is shown in Table 703 2 .

Table 703—Format of the AAI_L2_XFER message

Management Message Type = TBD

Transfer-Type

Sub-Type

Type specific message payload

The enumeration of Transfer-Type is as follows:

a) Transfer-Type = 1; GNSS assistance (DL)

b) Transfer-Type = 2; LBS measurement [Terrestrial meas. and GNSS pseudo ranges] (UL)

c) Transfer-Type = 3; Device Bootstrap (DL/UL)

d) Transfer-Type = 4; WirelessMAN-OFDMA network boundary indication (DL)

e) Transfer-Type = 5; ORAT-MSG (DL)

a)  Sub-Type = 1: GERAN (GSM/GPRS/EGPRS)

b)  Sub-Type = 2: UTRAN

c)  Sub-Type = 3: E-UTRAN

d)  Sub-Type = 4: TDSCDMA

e)  Sub-Type = 5: CDMA2000

f)      Transfer-Type = 6: SMS

a)     Sub-Type = SMS data

b)    Sub-Type = SMS confirmation

g)     Transfer-Type = 7: MIH Frame

i)     Sub-Type =1 : ES/CS MIH Capability Discovery

ii)    Sub-Type =2 : Event Service

iii)   Sub-Type = 3 : Command Service

iv)   Sub-Type = 4 : Information Service

h) Transfer-Type = 8; ASN control messages for Relay support (DL/UL)

h i) Transfer-Type = 8-127; reserved

i j) Transfer-Type = 128-255; Vendor specific types

Part-II: Changes to Relay section 16.6 to support distributed control

[Note to Editor: Modify the text in section 16.6.1 to 16.6.2.13 as follows]

16.6 Support for Relays

16.6.1 Relay Modes and General Description

In Advance WirelessMAN-OFDMA System, support for relay is an optional feature. The AMSs may associate either with an ARS or an ABS and receive services from the ARS or ABS that they are attached to.

Relaying in Advance WirelessMAN-OFDMA System is performed using a decode and forward paradigm. Both, TDD and FDD modes for Duplexing the DL and UL are supported. In TDD deployments ARSs operate in TTR mode, whereby the access and relay link communications are multiplexed using time division multiplexing within a single RF carrier.

In Advance WirelessMAN-OFDMA System, the ARSs operate in non-transparent mode which essentially means that

(a) the ARSs compose the SFH and the A-MAPs for the subordinate stations

(b) the ARSs transmit the A-Preamble , SFH and A-MAPs for the subordinate stations.

In Advance WirelessMAN-OFDMA deployment supporting relay, a distributed scheduling model is used where in each infrastructure station (ABS or ARS) schedules the radio resources on its subordinate link. In case of an ARS, the scheduling of the resources is within the radio resources assigned by the ABS. The ABS notifies the ARSs and AMSs of the frame structure configuration. The radio frame is divided into access and relay zones as described in section 16.6.3.1.

In the access zone, the ABS and the ARS transmit to, or receive from, the AMSs. In the relay zone, the ABS transmits to the ARSs and the AMSs, or receives from the ARSs and AMSs. The start times of the frame structures of the ABS and ARSs are aligned in time. The ABS and ARSs transmit A-Preamble, SFH, and A-MAPs to the AMSs at the same time.

The ABS may exercise control over the manner in which ARSs utilize the resources within the access zone.

16.6.2 MAC access control

16.6.2.1 Addressing

16.6.2.1.1 Station Identifier (STID)

The ABS assigns an STID to the ARS during network entry. The STID uniquely identifies the ARS within the domain of the ABS. The structure of the STID is described in section 16.2.1.2.1.

16.6.2.1.2 Flow Identifier (FID)

An FID, unique within the ARS, is assigned to each connection on the ABS-ARS link. The structure of the FID is described in section 16.2.1.2.2.

The prime function of an ARS is to relay MAC PDUs between the ABS and the AMS, for which one or more connections may be established between the ABS and the ARS. Only the control connections, meant for transporting control messages between the ABS and the ARS, terminate at the ARS.

16.6.2.2 MPDU Formats

MAC PDUs for AMSs sent on the relay connections on the relay link shall be encapsulated into a relay MAC PDU. The format of the relay MAC PDU is illustrated in Figure 576.

Figure 576 Relay MAC PDU format

Each relay MAC PDU shall begin with a Generic MAC header. The format of GMH is same as defined in section 16.2.2.1.1. The GMH may be followed by one or more extended headers. The relay MAC PDU shall also contain a payload. The payload consists of one or more AMSs’ MAC PDUs.

MAC PDUs for the ARS sent on the control connections on the relay link shall follow the same format as shown in Figure 385.

16.6.2.2.1 STID Extended header

The STID Extended Header is inserted into the Relay MAC PDU in order to carry the STID of the AMSs whose PDUs are encapsulated into the Relay MAC PDU. The extended header carries one STID for each PDU encasulated in the Relay MAC PDU.The format of STID Extended Header is defined in Table 940—.

Table 940 - STID Extended Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| STID Extended header() { |  |  |
| Last | 1 | Last Extended Header indication: 0 = one or more extended header follows the current extended header unless specified otherwise; 1 = this extended header is the last extended header unless specified otherwise |
| Type | TBD | STID EH |
| Do{ |  |  |
| STID | 12 | Identity of the AMS, which tansmits or receives MAC PDU. |
| End | 1 | 1: This STID is the last STID; 0: one or more STID follows. |
| }While(!End) |  |  |
| } |  |  |

16.6.2.2.2 Length Extended header

The length extended header (LEH) is added to relay MAC PDU when the relay MAC PDU length is greater that 2048 bytes. The LEH if present in the relay MAC PDU, shall be the first extended header in the relay MAC PDU. The format of LEH is defined in Figure 941. The Length field in LEH gives the 3 MSBs of 14 bit extended length of relay MAC PDU. The length field in GMH gives the 11 LSBs of 14 bit extended length of relay MAC PDU.

Figure 941 – Length Extended header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Length Extended header() { |  |  |
| Last | 1 | Last Extended Header indication: 0 = one or more extended header follows the current extended header unless specified otherwise; 1 = this extended header is the last extended header unless specified otherwise |
| Type | TBD | Length EH |
| Length | 3 | 3 MSB length fields to indicate the length of relay MAC PDU. The 11 LSBs of Length field exist in the GMH. |
| } |  |  |

16.6.2.3 Construction and Transmission of MPDUs

16.6.2.3.1 Concatenation

In Multi-Hop Relay networks, multiple MAC PDUs for an ARS and multiple relay MAC PDUs can be concatenated into a single transmission on the relay link.

16.6.2.3.2 Data Forwarding Scheme

For DL transmission via ARS, when an ABS transmits data to AMSs via an ARS, the ABS shall encapsulate MAC PDUs of one or multiple target AMSs into a relay MAC PDU of ARS and appends an STID Extended header. The ARS shall decode the DL basic assignment A-MAP IE to receive relay MAC PDUs. The ARS shall de-encapsulate received relay MAC PDU and transmits the MAC PDUs to target AMSs based on STID in STID Extended header.

For UL transmission via ARS, The ARS shall encapsulate the MAC PDUs from one or multiple AMSs into a relay MAC PDU and appends an STID Extended header. The ABS decodes the received relay MAC PDU and extracts MAC PDUs for each AMS.

16.6.2.3.3 Forwarding control messages between the RS and the ASN

Upon ABS receiving the downlink control messages from the ASN, it performs classification to recognize that it is an ARS related control message from the ASN. Then the ABS translates the control message between the two interfaces by means of encapsulating the control message in a “AAI_L2-XFER” MAC management message  and sends it to the target ARS with FID=1. In order to optimize the message size, the ABS may remove ASN transport network headers of the control message before transmitting the same to the ARS – the details of which are outside the scope of this specification.

On the uplink, the ARS sends the control message using AAI_L2-XFER message on FID=1. Upon ABS receiving the uplink control messages from the ARS, it attaches the ASN transport network headers and forwards the message to the ASN.

16.6.2.4 Security

ARS uses the same security architecture and procedures as an AMS to provide privacy, authentication and confidentiality between itself and ABS on the relay link.

An ARS is operating as distributed security mode. The AK established between an AMS and an authenticator is derived as follows and distributed to this ARS during key agreement.

AK = Dot16KDF(PMK, MSID*|RSID|CMAC_KEY_COUNT|”AK”, 160)

As shown in Fig 577, after authorization for AMS completes and the MSK for AMS is established, the ARS starts key agreement with the AMS. During the key agreement, the authenticator shall transfer the relevant Authorization Key (AK) context associated with the AMS to its ARS. On obtaining AK context the ARS derives necessary security keys such as CMAC keys and TEK from the AK and ARS is responsible for key management of AK, CMAC keys and TEK, and interacts with the AMS as if it were an ABS in the AMS’s perspectives.

During the key agreement, similarly to macro BS, the Security Association shall be established between an AMS and an ARS. ARS uses the set of active keys shared with the AMS to perform encryption/decryption and integrity protection on the access link.

The ARS runs a secure encapsulation protocol with the ABS based on the primary SA, which is established between ARS and ABS. The access RS uses the set of active keys shared with ABS to perform encryption/decryption and integrity protection on the relay link.

MPDUs are encapsulated into one R-MPDU and en/decrypted at once by primary SA, which is established between ARS and ABS.

The security context used for relay link (between ABS and ARS) and access links (between ARS and AMS) are different and maintained independently. The key management follows as the same method as Macro ABS defined in Section 16.2.5.2.

Figure 577- Key agreement procedure

16.6.2.5 Handover

16.6.2.5.1 Network topology advertisement

The ARS shall broadcast information about the neighbor ABSs/ARSs that are present in the network using the AAI_NBR-ADV message defined in 16.2.3.12<<15.2.6.10>>. The ARS may obtain the information to be included in the AAI_NBR-ADV message from its management plane serving ABS. Each ARS can broadcast a different AAI_NBR-ADV message that is suitable for its service area.

To facilitate each ARS to transmit an AAI_NBR-ADV message suitable for its service area, the ABS shall transfer neighbor list information to the ARSs over the relay links according to the specific neighborhood of the receiving ARS. In order to compose the neighbor list customized for the subordinate ARSs, the ABS may use the location information or the interference measurement reports received from the ARSs.

An ARS, depending on its capability and depending on the messages that it receives, may choose between one of the following options in generating the AAI_NBR-ADV message:

a) An ARS may broadcast the AAI_NBR-ADV message without modifying the received neighbor list

b) An ARS may further customize and compose an AAI_NBR-ADV message that is suitable for its service area by utilizing the received neighbor list as well as any additional information that the ARS itself may have obtained, for instance by scanning or measurement.

16.6.2.5.2 AMS scanning of neighbor ABSs/ARSs

The scanning follows the same procedure as described in Section 16.2.6.1.2, with the exception that when serving station is an ARS, the ARS defines the corresponding trigger/action, controls the scanning procedure and initiates handover based on received scanning report.

16.6.2.5.3 AMS Handover process

The handover process follows the same procedure as described in Section 16.2.6, with every instance of ABS replaced by ARS.

During handover, serving station (ABS or ARS) may exchange AMS context with the target station (ABS or ARS) for handover optimization using AAI_L2-XFER messages carrying ASN payloads. The target station shall allocate the STIDs and FIDs for the AMS, during handover. If the target station is an ARS, the STID and FID allocation happens as specified in the AMS network entry (16.6.2.10.1) and AMS connection management (16.6.2.6.1) sections respectively.

16.6.2.6 Scheduling and QoS

The ABS may use persistent allocations (as described in section 16.2.7) and group resource allocations (as described in section 16.2.9) on the relay link. ARSs shall support the use of persistent scheduling and group resource allocations on the relay link.

The ARS shall schedule air link resources on the access link for communications with its associated AMSs. Frame-by-frame scheduling decisions are made by the ARS. ARS may receive control information from the ASN that constrains ARS use of the air interface resources in a method outside of the scope the standard.

, however the ABS shall have the ability to constrain the resources utilized by the ARS. The ABS may specify the following resource usage constraints:

- Frequency partitions or subbands which RS is allowed to use/not allowed to use.

- Tx power that RS should use in a given partition or subband.

- Restrict/recommend groups of PMIs that RS can use in specified subbands

16.6.2.6.1 Connection management

ARS controls the connection management for the associated AMS, i.e., AAI_DSx messages shall be terminated at the ARS. ARS performs FID assignments for the AMS. ARS communicates with the other network entities in the Data path (eg: ABS, ASN entities etc) using AAI_L2-XFER messages carrying ASN payloads to complete the data path setup for this FID of the AMS..

16.6.2.7 Bandwidth Request and Grant Management

ARSs directly handle the bandwidth requests that they receive from associated AMSs. An ARS may receive bandwidth requests from its associated AMSs via any of the mechanisms described in section <<165.2.11.1>>. ARSs shall handle all bandwidth requests from AMSs locally, using the same protocol as ABSs (as described in section <<165.2.11>>).

ARSs may request uplink bandwidth from the ABS using one of the BW request mechanisms defined in section <<165.2.11>>. An ARS shall request bandwidth using the FID of one of the connections established between itself and the ABS. The ARS may request bandwidth for multiple connections using a single bandwidth request.

The bandwidth grant messages and procedures defined in section <<165.2.11.2>> shall be used by the ARS and AMS on the access link and between the ABS and ARS on the relay link.

16.6.2.8 ARQ

The ARS performs ARQ operation with ABS in the relay link and AMS in the access link.

In downlink, ABS generates and sends an ARQ block to ARS in the relay link. If the ARQ block is corrupted in the relay link, ARS shall send a NACK to ABS, and then ABS shall prepare and perform retransmission. If ARS receives the ARQ block correctly in the relay link, ARS shall forward the ARQ block to the AMS in the access link. In the access link, if the ARQ block is corrupted and the ARS receives a NACK from AMS, ARS shall prepare and perform retransmission to AMS.

In uplink, AMS generates and sends ARQ block to ARS in the access link. If the ARQ block is corrupted in the access link, ARS sends a NACK to AMS, and then AMS shall prepare and perform retransmission. If ARS receives the ARQ block is correctly in the access link, ARS shall forward ARQ block to the ABS. In the relay link, if the ARQ block is corrupted and ARS receives a NACK from ABS, ARS shall prepare and perform retransmission.

16.6.2.9 HARQ

ARS shall perform HARQ operation with ABS in the relay link and AMS in the access link independently. The HARQ signaling and timing protocol between with adjacent station (ABS and ARS or ARS and AMS) follows the section 16.2.14.2.

16.6.2.9.1 Group resource allocation HARQ Signaling and Timing at ARS

ARS shall perform Group resource allocation with ABS in the relay link and AMS in the access link independently. The HARQ signaling and timing protocol between with adjacent stations (ABS and ARS or ARS and AMS) follows the section 16.2.14.3 <<15.3.9.3>>.

16.6.2.9.2 Persistent allocation HARQ Signaling and Timing at ARS

ARS shall perform persistent resource allocation with ABS in the relay link and AMS in the access link independently. The HARQ signaling and timing protocol between with adjacent stations (ABS and ARS or ARS and AMS) follows the section 16.2.14.43.9.4.

16.6.2.10 Network Entry

16.6.2.10.1 AMS Network Entry

In DL channel scanning phase, an AMS may select an ARS as its access station. The AMS shall establish synchronization with the ARS and obtain DL/UL parameters by reading SFHs and AAI_SCD message from the ARS.

After ranging procedure described in subclause <<15.3.10.3.1>>16.2.15.3, the ARS and AMS shall finish capability negotiation, authentication/key agreement and registration procedures. The ARS communicates with the ASN entities using AAI_L2-XFER messages carrying ASN payloads, during the AMS network entry. The ARS assigns the STID to the AMS. The mechanism for ensuring uniqueness of the STID in the ABS domain is outside the scope of the specification. The ABS is notified by the ARS of the assigned STID to the AMS via ASN messaging. the ARS shall relay the management messages for the remaining network entry procedures i.e., AAI_SBC-REQ, AAI_SBC-RSP, AAI_PKM-REQ, AAI_PKM-RSP, AAI_REG-REQ and AAI_REG-RSP messages between the AMS and the ABS.

16.6.2.10.2 ARS Network Entry

16.6.2.10.2.1 Relay station network entry

The network entry and initialization for relay station follows the procedures defined in 16.2.16. In addition, after the Capability exchange and registration phase, the ARS shall perform 2 addition procedures (phases g and h listed below), i.e. the ARS also obtains neighbor station measurement report and configuration of the operational parameters as indicated below:

a) Scan for DL channel and establish synchronization with the ABS

b) Obtain DL/UL parameters (from SuperFrameHeader)

c) Perform ranging

d) Basic capability negotiation

e) Authorization, authentication, and key exchange

f) Registration with ABS and ASN

g) Obtain neighbor station measurement report

hg) Configure operational parameters

The procedure for initialization of an ARS shall be as shown in Figure 578.

Figure 578 – ARS initialization overview

16.6.2.10.2.1.1 ABS and ARS behavior during scanning and synchronization

An ARS shall follow the same scanning and synchronization procedure as that specified for an AMS. In addition, the ARS may store SA-PREAMBLE and corresponding signal strength information in order to report the stored values to the ABS after registration, by sending an AAI_NBR-MEAS-REP message during the neighbor station measurement report phase upon request. The ABS indicates this request for the information through ‘Neighbor station measurement report indicator’ in the AAI_RNG-RSP message during initial ranging.

16.6.2.10.2.1.2 ARS neighbor station measurement report

The ABS may request the ARS to do neighbor measurements during network entry using the ‘Neighbor station measurement report indicator’ in the AAI_RNG-RSP message. The neighbor station measurement report includes the signal strength and SA-PREAMBLE of neighbor ABS(s) or ARS(s).

If the measurement is not required, after registration the ARS shall skip the neighbor station measurement report phase and go to the ARS operation parameter configuration phase.

The ARS may obtain the neighbor monitoring scheme parameters from a configuration command message. After the configuration command message is sent to an ARS to inform it of the monitoring scheme, the ABS shall not change the frame structure configuration before the ARS enters into operational mode. The ARS shall use the available neighborhood measurement mechanisms defined in 16.6.xx, 16.6.(xx+1), and 8.4.6.1.1.4.

The ARS shall then send the measurements to the ABS using the AAI_NBR-MEAS-REP message. This measurement report is used by the ABS to assign the ARS a SA-PREAMBLE based on the report from the ARS.

16.6.2.10.2.2 Interference measurement in Relay station

In order to predict the interference or SINR of the radio links for different topology and radio resource reuse pattern, the following prediction method may be considered based on the RSSI reported by AAI_NBR-MEAS-REP message (see 16.2.5.bb).

16.6.2.10.2.3 Relay station location information

In order to assist ARS neighborhood discovery, an ABS may send an AAI_LOC-REQ message to an ARS. Upon receiving the AAI_LOC-REQ message, the ARS shall report its location information by sending an AAI_LOC-RSP message to the ABS.

In order to obtain the location information of neighbor stations, an ARS may send an AAI_LOC-REQ message to the ABS. Upon receiving the AAI_LOC-REQ message, the ABS shall report the location information of neighboring stations by sending an AAI_LOC-RSP message to the ARS.

The message sequence charts (Figure a, Figure b) describe the Relay station location request and report that shall be followed by compliant ARSs and ABSs.

Figure a - Relay station location report (ABS request)

Figure b - Relay station location report (ARS request)

16.6.2.10.2.24 Handing of AMS Initial Ranging by the ARS

ARS allocates its own ranging opportunities and ARS ranging channel configuration is carried in ARS’s SFH. ARS handles the ranging procedure. The initial ranging sequence is handled by the ARS itself, while the AAI_RNG-REQ message handling is partly done at the ARS and partly at the ABS.

When the ARS detects a ranging sequence on the access link, the adjustments related to physical ARS-AMS link, if necessary, are performed by the ARS directly with the AMS (without any interaction with the ABS).

When the ARS receives the AAI_RNG-REQ message associated with initial network entry of AMS with an MS_Random with the MCRC masked by 12-bit RAID for Ranging, it assigns the TSTID to the AMS and responds back with AAI_RNG-RSP message to AMS. The mechanism for ensuring uniqueness of the TSTID in the ABS domain is outside the scope of the specification. forward the AAI_RNG-REQ after removing the parameters that the ARS itself manages. If the AAI_RNG-REQ message from the AMS, forwarded by the ARS, is accepted by the ABS, the sends an AAI_RNG-RSP message to the ARS/ After that, the access ARS shall fill the AAI_RNG-RSP with the parameters that the ARS itself manages and transmit the AAI_RNG-RSP to the AMS.

When an ARS wishes to perform initial ranging with an ABS, the ARS shall follow the same steps as an AMS would, when the AMS performs initial ranging with the ABS as described in section <<15.3.10.3.1>>16.2.15.3.

16.6.2.11 Ranging

16.6.2.11.1ARS Initial Ranging

When an ARS wishes to perform initial ranging with an ABS, the ARS shall follow the same steps as an AMS would, when the AMS performs initial ranging with the ABS as described in section <<15.3.10.3.1>>16.2.15.3.

16.6.2.11.2 Handling of AMS and ARS Periodic Ranging handling by the ARS

When an ARS detects a periodic ranging sequence on the access link, it shall perform adjustments directly with the AMS with no interaction with the ABS. The ARS decides on the appropriate adjustments if required.

When an ARS initiates a periodic ranging on the relay link to the ABS, the ARS shall perform the same tasks as an AMS performs with the ABS.

The ranging channel for synchronized AMSs, as described in section 16.3.9.1.4.2 is also used for ARS periodic ranging.

16.6.2.12 Sleep Mode

When an AMS is attached to an ARS for idle mode operation, procedures defined in section 16.2.16 where each instances of ABS is replaced by ARS.

16.6.2.13 Idle Mode

When an AMS is attached to an ARS for idle mode operation, procedures defined in section 16.2.17 where each instances of ABS is replaced by ARS.