| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for MSs in Relay Network (General description) (15.6.2) |  |
| Date Submitted | 2009/11/6 |  |
| Source(s) | Yoshikazu Watanabe, Yasuhiro Mizukoshi, Linghang Fan, Nader Zein, Tetsu Ikeda NEC | Voice: +81443963530 E-mail: y-watanabe@fa.jp.nec.com |
| Re: | LB comment to 802.16m Amendment Working Document D2 P802.16m/D2 Relay |  |
| Abstract | This contribution proposes text for the Draft Amendment regarding support for Legacy MS in Relay Network. |  |
| Purpose | To be discussed and adopted by TGm for the P802.16m/D3 Amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support for Legacy MS in Relay Network (General description) (15.6.2)

Yoshikazu Watanabe, Mizukoshi Yasuhiro, Linghang Fan, Nader Zein, Tetsu Ikeda

NEC

# Introduction

This contribution proposes text for the Draft Amendment [1] regarding support for MSs in Relay Network.

# Proposed Text

-------------------------------------------------- Start of the proposed Text --------------------------------------------------

# 15.6 Support for Relay

# 15.6.2 MAC access control

# 15.5.2.16 Support for MSs

An ARS that is capable of supporting the WirelessMAN-OFDMA MSs, communicates with MSs in the LZone. An ABS that is capable of supporting the Wireless MAN-OFDMA MSs via ARSs, communicates with ARSs in the MZone.

When a Wireless MAN-OFDMA MS performs network entry procedure via an ARS, the ABS shall assign a Basic CID and a Primary Management CID to the MS for the communication in the LZone. In addition, the ABS shall assign an STID and two FIDs to the MS for the communication in the MZone. When the MS establishes a new connection, the ABS shall assign an FID to the connection in addition to a CID. The ABS and the ARS shall keep the mapping between the CID and the combination of the STID and the FID.

The ABS and the ARS shall use the STID and FIDs to identify the MS and connections for the MS in MAC PDUs and management messages communicated over the MZone. MAC PDUs with data payload to/from the MS are transmitted in the form of the AAI MAC PDU format in the MZone. The ABS and the ARS shall transform the MAC PDUs between the AAI MAC PDU format and the legacy MAC PDU format when processing and relaying them.

# 15.5.2.16.1 Addressing

ABS shall assign an STID to each legacy MS. The STID assigned to an MS shall be greater than 0 and less than or equal to ‘m’.

ABS shall assign an FID to each connection between the ABS and a legacy MS. ABS shall use FID 0 and 1 for a Basic CID and a Primary Management CID, respectively. ABS shall not use FID 0 and 1 for a data connection.

When a CID is assigned to a connection, ABS shall derive the CID from the STID and the FID assigned to the connection in the way specified in section 15.5.2.16.2.

If Seamless HO is enabled, ‘a’ shall be less than or equal to 14.

# 15.5.2.16.2 Mapping between CID and STID/FID

ABS and ARS shall map between an CID and the combination of an STID and an FID by numerical operation.

ABS and ARS shall derive the CID corresponding to an STID and an FID as follows.

CID = STID + ( FID･m )				(if FID <= 1)

CID = STID･16 + FID + 2m + 1			(if FID > 1)

where m is the value of the system parameter ‘m’.

ABS and ARS shall derive the STID and the FID corresponding to a CID as follows.

STID = ( ( CID - 1 ) mod m ) + 1			(if CID <= 2m)

STID = floor( ( CID - 2m - 1) / 16 ) 			(if CID > 2m)

FID = floor( ( CID - 1) / m )				(if CID <= 2m)

FID = ( CID - 2m - 1) mod 16			(if CID > 2m)

# 15.5.2.16.3 MAC PDU transformation

MAC PDU transformation from the legacy MAC PDU format to the AAI MAC PDU format by a station (ABS or ARS) shall be done as follows.

- The station extracts original payload (one or more MAC SDUs and/or SDU fragments, or a MAC management message) from the legacy MAC PDU. MAC subheaders and extended subheaders in the legacy MAC PDU is processed by the station.

- The station prepares an AGMH and an FPEH/FEH for the original payload. The FID field of the AGMH is set to the FID corresponds to the CID in the GMH of the legacy MAC PDU.

- The station constructs an AAI MAC PDU from the AGMH and the FPEH/FEH and the original payload. The STID for the AAI MAC PDU is the STID corresponds to the CID in the GMH of the legacy MAC PDU.

The station may construct one AAI MAC PDU from multiple legacy MAC PDUs which belong to the same connection. The station may construct multiple AAI MAC PDUs from one or more legacy MAC PDU(s) which belong to the same connection. The station may add and/or change boundaries for fragmentation.

MAC PDU transformation from the AAI MAC PDU format to the legacy MAC PDU format by a station (ABS or ARS) shall be done as follows.

- The station extracts original payload (one or more MAC SDUs and/or SDU fragments, or a MAC management message) from the AAI MAC PDU. Extended headers in the AAI MAC PDU is processed by the station.

- The station prepares an legacy GMH and FSH(s)/PSH(s) for the original payload. The CID field of the legacy GMH is set to the CID corresponds to the STID and the FID of the AAI MAC PDU.

- The station constructs an legacy MAC PDU from the legacy GMH and the FSH(s)/FEH(s) and the original payload.

The station may construct one legacy MAC PDU from multiple AAI MAC PDUs which belong to the same connection. The station may construct multiple legacy MAC PDUs from one or more AAI MAC PDU(s) which belong to the same connection. The station may add and/or change boundaries for fragmentation. The station may add MAC subheaders and/or extended subheaders to the legacy MAC PDU.

-------------------------------------------------- End of the proposed Text ---------------------------------------------------

# References

[1]	IEEE P802.16m/D2 Draft