| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_SCN-REQ/RSP messages (15.2.3.12/15.2.3.13) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057: IEEE 802.16 Working Group Letter Ballot Recirc #30a Section 15.2.3.12 AAI_SCN-REQ, 15.2.3.13 AAI_SCN-RSP |  |
| Abstract | This contribution provides updated the AAI_SCN-REQ/RSP messages. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_SCN-REQ/RSP messages (15.2.3.12/15.2.3.13)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

AMS cannot scan neighbor ABS only with 48-bit BSID in the AAI_SCN-RSP message; it requires preamble index if the ABS is not in the AAI_NBR-ADV message. Moreover, Table 681 and 682 in D2 has several editorial errors.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

{Replace Table 681 and Table 682 with the following two tables.}

15.2 Medium access control

15.2.3.12 AAI_SCN-REQ

Table 681—Parameters for AAI_SCN-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| Scan purpose | 0x0: scan BS(s) which is in the list of the AAI_NBR-ADV message 0x1: scan BS(s) which is not in the list of the AAI_NBR-ADV message |  |
| Scan duration | Duration (in units of subframes) of the requested scanning period |  |
| Interleaving Interval | The period of AMS's normal operation which is interleaved between Scan Durations. |  |
| Scan Iteration | The requested number of iterating scanning interval by an AMS. |  |
| Recommended Start Frame indication | Indicates whether Recommended start super frame number field and Recommended start frame field are included |  |
| N_Recommended_BS_ Index_indication | Indicates whether N_Recommended_BS_Index field is included in this message |  |
| N_Recommended_BS_ Full_indication | Indicates whether N_Recommended_BS_Full field is included in this message. |  |
| Recommended start super frame number | Represents the 6 least significant bits of the absolute super frame number for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value. | Shall be included when Recommended Start Frame indication is set to 1 |
| Recommended start frame number | Represents recommended start frame number within the super frame number which is indicated by Recommended start super frame number field. The value indicates: 0b00: The 1st frame in the super frame 0b01: The 2nd frame in the super frame 0b10: The 3rd frame in the super frame 0b11: The 4th frame in the super frame | Shall be included when Recommended Start Frame indication is set to 1 |
| N_Recommended_BS_ Index | This is the number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. When an AMS receives AAI_SCN-RSP message from BS in response to AAI_SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI_SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI_SCN-REQ message to the ABS. | Shall be included when N_Recommended_BS_ Index_indication is set to 1 |
| Configuration Change Count for AAI_NBR-ADV | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID |  |
| Neighbor_BS_Index | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Shall be included when N_Recommended_BS_Index > 0 |
| N_Recommended_ BS_Full | Number of neighboring BS to be scanned, which are using full 48-bit BSID. | Shall be included when N_Recommended_BS_Full_indication is set to 1 |
| Recommended BS ID | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_BS_Full > 0 |
| N_Recommended_ Carrier_Index | Number of carriers to be scanned at each neighboring ABS. | May be included when N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| Recommended_Carrier_ Index | Recommended carrier index for scan at each neighboring ABS. | Shall be included when N_Recommended_Carrier_Index > 0 |
| N_Recommended_ Carrier_Index_ at_Serving_ABS | Number of logical carrier index of the serving ABS to be scanned | Carrier index at serving ABS |
| Recommended logical carrier index to be used for scanning | Shall be included when N_Recommended_Carrier_Index_ at_Serving_ABS > 0 |  |

Table 682—Parameters for AAI_SCN-RSP message

| Name | Value | Usage |
| --- | --- | --- |
| Scan purpose | 0x0: scan BS(s) which is in the list of the AAI_NBR-ADV message 0x1: scan BS(s) which is not in the list of the AAI_NBR-ADV message |  |
| Scan duration | Duration (in units of subframes) of the requested scanning period |  |
| Report mode | 0b00:No report 0b01:Periodic report 0b10:Event-triggered report 0b11:One-time scan report |  |
| Report metric | Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4-5: Reserved; shall be set to zero |  |
| Report period | The period of MS's report of scanning result when the MS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the BS sends an unsolicited AAI_SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the MS receives the AAI_SCN-RSP message. For MS scanning request denied by AAI_SCN-RSP with Scan Duration set to zero, Report period is the number of frames that BS suggests to MS before transmitting next AAI_SCN-REQ |  |
| N_Recommended_BS_ Index_indication | Indicates whether N_Recommended_BS_Index field is included in this message. This is valid only if Scan duration is not equal to zero. |  |
| N_Recommended_BS_ Full_indication | Indicates whether N_Recommended_BS_Full field is included in this message. This is valid only if Scan duration is not equal to zero. |  |
| Start super frame number | Represents the 6 least significant bits of the absolute frame number in which the first Scanning interval starts | Included only when Scan Duration > 0 |
| Start frame number | Represents start frame number in the Start super frame number. 0b00: The 1st frame in the super frame 0b01: The 2nd frame in the super frame 0b10: The 3rd frame in the super frame 0b11: The 4th frame in the super | Included only when Scan Duration > 0 |
| Interleaving Interval | The period of AMS's normal operation which is interleaved between Scan Durations. | Included only when Scan Duration > 0 |
| Scan Iteration | The requested number of iterating scanning interval by an AMS. | Included only when Scan Duration > 0 |
| N_Recommended_BS_ Index | Number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. | Included only when Scan Duration > 0 |
| Configuration Change Count for AAI_NBR-ADV | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID | Included when N_Recommended_BS_Index > 0 |
| Neighbor_BS_Index | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Shall be included when N_Recommended_BS_Index > 0 |
| N_Recommended_ BS_Full | Number of neighboring BS to be scanned, which are using full 48-bit BSID. | Included when Scan Duration > 0 |
| Recommended BS ID | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_BS_Full > 0 |
| Preamble Index of Recommended BS ID | Preamble Index of Recommended BS ID | Shall be included when N_Recommended_BS_Full > 0 |
| N_Recommended_ Carrier_Index | Number of carriers to be scanned at each neighboring ABS. | May be included for a multi-carrier target ABS |
| Recommended_Carrier_ Index | Recommended carrier index for scan at each neighboring ABS. | Included when N_Recommended_Carrier_Index > 0 |
| Carrier index at serving ABS | Recommended logical carrier index to be used for scanning | May be included when the serving ABS is a multi-carrier ABS |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------