| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_SCN-REQ/RSP messages (15.2.3.12/15.2.3.13) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057: IEEE 802.16 Working Group Letter Ballot Recirc #30a Section 15.2.3.12 AAI_SCN-REQ, 15.2.3.13 AAI_SCN-RSP |  |
| Abstract | This contribution provides updated the AAI_SCN-REQ/RSP messages. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_SCN-REQ/RSP messages (15.2.3.12/15.2.3.13)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

AMS cannot scan neighbor ABS only with 48-bit BSID in the AAI_SCN-RSP message; it requires preamble index if the ABS is not in the AAI_NBR-ADV message. Moreover, Table 681 and 682 in D2 has several editorial errors.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

{Replace Table 681 and Table 682 with the following two tables.}

15.2 Medium access control

15.2.3.12 AAI_SCN-REQ

Table 681—Parameters for AAI_SCN-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| . . . |  |  |
| N_Recommended_BS_ Index_indication | Indicates whether N_Recommended_BS_Index field is included in this message |  |
| N_Recommended_BS_ Full_indication | Indicates whether N_Recommended_BS_Full field is included in this message. |  |
| N_Recommended_Preamble_ Index_indication | Indicates whether N_Recommended_Preamble_Index field is included in this message |  |
| . . . |  |  |
| N_Recommended_ BS_Full | Number of neighboring BS to be scanned, which are using full 48-bit BSID. | Shall be included when N_Recommended_BS_Full_indication is set to 1 |
| Recommended BS ID | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_BS_Full > 0 |
| N_Recommended_ Preambe_Index | Number of preambles to be scanned, | Shall be included when N_Preamble_Index_Indication is set to 1 |
| Preamble Index | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_Preamble_Index> 0 |
| . . . |  |  |

Table 682—Parameters for AAI_SCN-RSP message

| Name | Value | Usage |
| --- | --- | --- |
| . . . |  |  |
| N_Recommended_ BS_Full | Number of neighboring BS to be scanned, which are using full 48-bit BSID. | Included when Scan Duration > 0 |
| Recommended BS ID | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_BS_Full > 0 |
| Preamble Index of Recommended BS ID | Preamble Index of Recommended BS ID | Shall be included when N_Recommended_BS_Full > 0 |
| . . . |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------