| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_SCN-REP message (15.2.3.14) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057: IEEE 802.16 Working Group Letter Ballot Recirc #30a Section 15.2.3.14 AAI_SCN-REP |  |
| Abstract | This contribution provides updated the AAI_SCN-REP message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_SCN-REP message (15.2.3.14)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

When an AMS reports scanning results, it may not know BSID information; it may only know about preamble index. Scanning report should be able to be done with preamble index only. Moreover, Table 6832 in D2 has several editorial errors.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

{Replace Table 683 with the following table.}

15.2 Medium access control

15.2.3.14 AAI_SCN-REP

Table 683—Parameters for AAI_SCN-REP message

| Name | Value | Usage |
| --- | --- | --- |
| Report mode | 0b00:No report 0b01:Periodic report 0b10:Event-triggered report 0b11:One-time scan report |  |
| Report metric | Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. For each bit location, a value of 0 indicates the trigger metric is not included, while a value of '1' indicates the trigger metric is included in the message. The bitmap interpretation for the metrics shall be: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4-5: Reserved; shall be set to zero |  |
| N_Neighbor_BS_Index_ indication | Indicates whether N_Recommended_BS_Index field is included in this message. |  |
| N_Neighbor_BS_Full_ indication | Indicates whether N_Recommended_BS_Full field is included in this message. |  |
| N_Scanned_Preamble_ Indices_indication | Indicates whether N_Scanned_Preamble_Indices field is included in this message. |  |
| BS CINR mean | The BS CINR Mean parameter indicates the CINR measured by the AMS from the serving ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment and averaged over the measurement period. | Included when Report metric Bit 0 = 1 |
| BS RSSI mean | The BS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the serving ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. The measurement shall be performed on the frame preamble and averaged over the measurement period. | Included when Report metric Bit 1 = 1 |
| Relative delay | This parameter indicates the delay of neighbor DL signals relative to the serving BS, as measured by the MS for the particular BS. The value shall be interpreted as a signed integer in units of samples. | Included when Report metric Bit 2 = 1 |
| BS RTD | The BS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the serving ABS. RTD can be given by the latest time advance taken by MS. The value shall be interpreted as an unsigned byte with units of 1/Fs. | Included when Report metric Bit 3 = 1 |
| N_Neighbor_BS_Index | Number of neighboring BS reported in this message and which are included in AAI_NBR-ADV message. | Included when N_Neighbor_BS_Index_indication == 1 |
| Configuration Change Count for AAI_NBR-ADV | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID | Included when N_Neighbor_BS_Index_indication == 1 |
| Neighbor_BS_Index | Based on the value of N_Neighbor_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Included when N_Neighbor_BS_Index > 0 |
| N_Neighbor_BS_Full | Number of neighboring BS reported in this message that are using full 48 bits BS ID. | Included when N_Neighbor_BS_Full_indication == 1 |
| Neighbor_BSID | Based on the value of N_Neighbor_BS_Full, full BSID (48-bit) is included. | Included when N_Neighbor_BS_Full > 0 |
| N_Scanned_Preamble_ Indices | Number of scanned preambles reported in this message | Included when N_Scanned_Preamble_Indices_indication == 1 |
| Preamble Index | Based on the value of N_Scanned_Preamble_Indices, preamble index (10-bit) is included. | Included when N_Scanned_Preamble_Indices > 0 |
| Scanning report of each recommended carrier at each neighboring ABS |  | May be included for multi-carrier target ABSs |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------