| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_SCN-REP message (15.2.3.14) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057: IEEE 802.16 Working Group Letter Ballot Recirc #30a Section 15.2.3.14 AAI_SCN-REP |  |
| Abstract | This contribution provides updated the AAI_SCN-REP message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_SCN-REP message (15.2.3.14)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

When an AMS reports scanning results, it may not know BSID information; it may only know about preamble index. Scanning report should be able to be done with preamble index only. Moreover, Table 6832 in D2 has several editorial errors.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

{Replace Table 683 with the following table.}

15.2 Medium access control

15.2.3.14 AAI_SCN-REP

Table 683—Parameters for AAI_SCN-REP message

| Name | Value | Usage |
| --- | --- | --- |
| . . . |  |  |
| N_Neighbor_BS_Index_ indication | Indicates whether N_Recommended_BS_Index field is included in this message. |  |
| N_Neighbor_BS_Full_ indication | Indicates whether N_Recommended_BS_Full field is included in this message. |  |
| N_Scanned_Preamble_ Indices_indication | Indicates whether N_Scanned_Preamble_Indices field is included in this message. |  |
| . . . |  |  |
| N_Neighbor_BS_Full | Number of neighboring BS reported in this message that are using full 48 bits BS ID. | Included when N_Neighbor_BS_Full_indication == 1 |
| Neighbor_BSID | Based on the value of N_Neighbor_BS_Full, full BSID (48-bit) is included. | Included when N_Neighbor_BS_Full > 0 |
| N_FA_Index | Number of scanned FAs | Included when N_Scanned_Preamble_Indices_indication == 1 |
| for (i=0; i<N_FA_Index; i++) { |  |  |
| FA_Index | Scanned FA Index |  |
| N_Scanned_Preamble_ Indices | Number of scanned preambles reported in this message |  |
| for (j=0; j<N_Scanned_ Preamble_Indices; j++) { |  |  |
| Preamble_Index | Based on the value of N_Scanned_Preamble_Indices, preamble index (10-bit) is included. |  |
| Report_Value | Based on the value of Report metric. |  |
| } |  |  |
| } |  |  |
| . . . |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------