| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Updates on the AAI_HO-REQ/CMD message (15.2.3.9/15.2.3.10) |  |
| Date Submitted | 2009-11-15 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057: IEEE 802.16 Working Group Letter Ballot Recirc #30a Section 15.2.3.10 AAI_HO-CMD |  |
| Abstract | This contribution provides updates on the AAI_HO-REQ/CMD message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Updates on the AAI_HO-REQ/CMD message (15.2.3.9/15.2.3.10)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

BSID in the AAI_HO-REQ and AAI_HO-CMD messages may not be included in the NBR-ADV message. For this case, preamble index and FA index need to be included in the AAI_HO-CMD messages.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

{Replace Table 679 and Table 680 with the following tables.}

15.2 Medium access control

15.2.3.9 AAI_HO-REQ

Table 679—Parameters for AAI_HO-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| Report Metric | Bit #0: BS CINR mean Bit #1: BS RSSI mean Bit #2: Relative delay Bits #3-6: Reserved; shall be set to zero. | Bitmap indicator of trigger metrics that the MS reports in this message. |
| N_New_BS_Index | Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message. |  |
| Neighbor_ABS_ID |  | Shall be included for each recommended neighbor BS Neighbor_ABS_ID may not be included in the AAI_NBR-ADV message. |
| ABS CINR mean |  | Shall be included if Report Metric Bit #0 is set to 1 |
| ABS RSSI mean |  | Shall be included if Report Metric Bit #1 is set to 1 |
| Relative delay |  | Shall be included if Report Metric Bit #2 is set to 1 |
| Carrier Preassignment Indication | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when Multi-Carrier Support = 1 |

15.2.3.10 AAI_HO-CMD

Table 680—Parameters for AAI_HO-CMD message

| Name | Value | Usage |
| --- | --- | --- |
| Mode | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. |  |
| HO Reentry Mode | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| HO Reentry Interleaving Interval | If HO Reentry Interleaving Interval > 0, the AMS continues data transmission with the Serving ABS within the HO Reentry Interleaving Interval and performs network reentry to the target ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB HO procedure per 15.2.9.2.9.2.2.1 | Shall be included when HO Reentry Mode is set to 1 |
| HO Reentry Iteration | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| Disconnect time offset | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Should be included when Mode = 0b00. |
| Resource_Retain_Time |  | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| Target ABS/YBS ID |  | Shall be included for each target ABS when Mode=0b00 |
| Preamble Index | Preamble Index of Target ABS | Shall be included for each target ABS when Mode=0b00 |
| FA index | FA Index of Target ABS | Shall be included for each target ABS when Mode=0b00 |
| S-SFH change count | The change count of this BS’s S-SFH IE. This value shall not be changed until ‘Ranging initiation deadline’ | Shall be included for each target ABS when Mode=0b00 |
| Action Time | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| CDMA_RNG_FLAG |  | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| Dedicated CDMA ranging code/opportunity |  | May be included when CDMA_RNG_FLAG=1 |
| HO process optimization | For each bit location, a value of 0 indicates the associated reentry management messages shall be required, a value of 1 indicates the reentry management message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP management messages during reentry processing Bit #1: Omit PKM Authentication phase except TEK phase during current reentry processing Bit #2: Seamless HO Bit #3: TBD. | Shall be included for each target ABS when Mode=0b00 |
| Ranging initiation deadline |  | Shall be included for each target ABS when Mode=0b00 |
| Service level prediction | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| Physical_Carrier_Index | Physical carrier index of the Target ABS where AMS will perform network reentry procedure. | Should be included for each target ABS which is multi-carrier ABS when Mode = 0b00 |
| Carrier Preassignment Indication | 1: information of pre-assigned secondary carriers by the target ABS is included in this message; 0: no pre-assigned secondary carrier | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| N_Preassigned_Carriers | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| Pre-assigned secondary carrier information | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| LZone Presambile Index | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| Pre allocated Basic CID | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------