| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the feedback polling A-MAP IE for multi-BS feedback (15.3.6.5.2.14) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Shuangfeng Han, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | Modifications are proposed for the feedback polling A-MAP IE for multi-BS feedback |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to the feedback polling A-MAP IE for multi-BS feedback (15.3.6.5.2.14)

David Mazzarese, Shuangfeng Han, Hokyu Choi

Samsung Electronics Co., Ltd.

# Introduction

Modifications are proposed for the feedback polling A-MAP IE for multi-BS MIMO feedback. The parameter CPI indicates the optional feedback of concatenating PMI (CPMI) for CL-MD or Co-MIMO operation (ICT = 0b10 and 0b11 respectively). In CL-MD and Co-MIMO operation, the AMS additionally always sends the feedback of the PMIs for the serving and neighboring cooperating ABSs. In operation without CPMI feedback, downlink coherent combining of the transmissions from the cooperating ABSs cannot be guaranteed. Since the same feedback of PMIs for the serving and neighboring cooperating ABSs could be obtained by operation with PMI coordination (ICT = 0b00 or 0b01), then there is no performance improvement by CL-MD or Co-MIMO if CPMI is not feedback. So we propose to delete the signaling of parameter CPI and makes the feedback of CPMI mandatory with CL-MD and Co-MIMO operation.

- References

[1] DRAFT amendment to IEEE standard for local and metropolitan area networks, P80216m/D2, Oct 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.6.5.2.14 Feedback Polling A-MAP IE” on line 19 page 397 as follows>

-------------------------------  Start of Text Proposal ---------------------------------------------------

15.3.6.5.4.12 Feedback Polling A-MAP IE

The information element shown in Table 821 is used by the ABS to schedule MIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

If the feedback includes only the quantized transmit correlation matrix when the ABS is equipped with 2 or 4 transmit antennas, or only the wideband information for MIMO feedback modes 0, 1, 4 and 7, or only the subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6, then the MS shall use an extended header. In other cases, for the feedback of the quantized transmit correlation matrix when the ABS is equipped with 8 transmit antennas, or for the feedback of subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6, or for multi-BS feedback, the MS shall use a MAC control message. In case of feedback for MIMO feedback modes 0, 1, 4 or 7, the MS shall feedback the CQI for FP0 if FPCT equals 1 or 4, or for FPk if FPCT is not equal to 1 or 4, where FPk is determined by k = floor(IDCell/255).

If MIMO_feedback_IE_type = 0b1, the AMS shall transmit using MIMO mode 0 if it has multiple transmit antennas, or using MIMO mode 1 with Mt=1 stream if it has a single transmit antenna. If MIMO_feedback_IE_type = 0b0, the AMS shall follow the MEF instruction in the IE.

Table 821 - Feedback Polling A-MAP IE

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| A-MAP IE Type { | [4] | Feedback_Polling_IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d ==0b000){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } else if (d != 0b000){ |  |  |
| Dedicated UL allocation | 1 | 0b0: No dedicated UL resource is allocated. BS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| If (Dedicated UL allocation ==0b1){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| } |  |  |
| Period (p) | 3 | Transmit feedback header every 4p frame |
| MIMO_feedback_IE_type | 1 | - 0b0: feedback for single-BS MIMO operation - 0b1: feedback for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | - Single-BS MIMO feedback request |
| If (Dedicated UL allocation == 0b1){ |  |  |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding with Mt = 2 if Nt=2 or 4, or Mt = 1 if Nt=1 - Non-adaptive precoding shall be used at the MS. Nt is the number of transmit antennas at the AMS. |
| } |  |  |
| Transmit_Correlation_Matrix | 1 | - 0b0: feedback of the transmit correlation matrix is indicated by CM - 0b1: feedback of the quantized BS transmit correlation matrix only - Transmit correlation matrix shall be feedback if CM = 0b1 |
| If (Transmit_Correlation_Matrix == 0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antenna Nt - If Nt=2 (SU-MIMO and MU-MIMO): - 0b0: 1 - 0b1: 2 - If Nt=4 (SU-MIMO and MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt=8 (SU-MIMO): - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - If Nt=8 (MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| MFM | 3 | - MIMO Feedback Mode for which the AMS shall feedback CQI and CSI to the ABS |
| If (MFM = 2, 3, 5, 6) { |  | - Feedback of CQI and CSI for localized resource units |
| Num_best_subbands | 3 | 0b000: report all subbands 0b001~0b111: Number of best subbands to report 1< Num_best_subbands ≤ Nsub |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | [TBD] | [TBD] |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| If(MFM=0, 1, 2, 5){ |  |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| } |  |  |
| } |  |  |
| Else { |  | - Multi-BS MIMO feedback request |
| TRU | 2 | - Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; With CL-MD and Co-MIMO, the AMS shall feedback Concatenating PMI (CPMI) for N_multiBS_reports-1 diversity set members. |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b10 or 0b11) { |  |  |
| CPI | 1 | Concatenating PMI Feedback indication; 0b0: feedback CPMI for N_multiBS_reports-1 diversity set members; 0b1: no feedback CPMI |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Reserved | 1 |  |
| Padding | variable | Padding to reach byte boundary |
| MCRC | 16 | 16 bit CRC masked by Station ID |

-------------------------------  End of Text Proposal  ---------------------------------------------------