| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Simultaneous Transmit and Receive (STR) Relay (15.6 Support for Relay) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Chanho Min, Heewon Kang, Youngbin Chang, Jungje Son, Rakesh Taori, Seunghee Han, Youngsoo Kim Samsung Electronics Co., Ltd. | Email: chanho.min@samsung.com yb.chang@samsung.com Phone: +82-31-279-5519 |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: Relay (Section 15.6) |  |
| Abstract | The contribution proposes the simultaneous transmit and receive (STR) relaying mode for 16m relay. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Simultaneous Transmit and Receive (STR) Relay

Chanho Min, Heewon Kang, Youngbin Chang, Jungje Son, Rakesh Taori, Seunghee Han, Youngsoo Kim

Samsung Electronics Co., Ltd.

- Introduction

This contribution includes the proposed text for the simultaneous transmit and receive (STR) relaying mode for Relay in the IEEE 802.16m AWD.

- Motivation

In 802.16j-2009, STR relay was already defined. STR relay has the following benefits compared with TTR relay.

- Increase spectral efficiency: Re-utilizes ABS-ARS link resources as ARS-AMS link resources

- Reduced overhead: No need for switching gap(R-TTI and R-RTI), relay amble, and Broadcasting information for frame configuration.

- Larger uplink coverage: Long TTI can be applied to whole UL frame which is not divided by Relay zone and Access zone.

- Minimize PHY change of ABS and AMS: ABS/ARS frame does not need to be divided into Access zone and relay zone. In PHY perspective, ABS transmits/receives data to/from ARS like an AMS, AMS transmits/receives data to/from ARS like an ABS.

- Text Proposal for STR relaying mode

-------------------------------------------------- Start of the proposed text --------------------------------------------------

[Add the following text after section 3.116 on page 4:]

3.117. Simultaneous transmit and receive (STR) relaying: a relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, and transmission to the superordinate station and reception from subordinate station(s) are performed simultaneously.

[Modify the following text from line 50-54 on page 561:]

Relaying in Advance WirelessMAN-OFDMA System is performed using a decode and forward paradigm. Both TDD and FDD modes for Duplexing the DL and UL are supported. In TDD deployments ARSs operate in either TTR mode, whereby the access and relay link communications are multiplexed using time division multiplexing within a single RF carrier, or STR mode, whereby the access and relay link communications are performed simultaneously.

-------------------------------------------------- End of the proposed text ---------------------------------------------------