| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on DL Pilot Patterns with one data stream (15.3.5.4.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | HanGyu Cho, Jinsoo Choi, Wookbong Lee LG Electronics | Voice: +82-31-450-1883 E-mail: hgcho@lge.com |
| Re | Comments on P802.16m/D2 for recirculation #30a |  |
| Abstract | The contribution proposes the text for DL pilot patterns with one data stream |  |
| Purpose | To be discussed and adopted into P802.16m/D2 by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on DL Pilot Patterns with one data stream (15.3.5.4.1)

HanGyu Cho, Wookbong Lee

LG Electronics

- Introduction

In the current P802.16m/D2 document, there are two pilot streams among which one is selected for one data stream transmission. Since the selection method is TBD, this contribution proposes the details on the selection method of pilot stream to be included in P802.16m/D2. The proposed text is developed so that it can be readily combined with IEEE 802.16-2009 [1], it is compliant to the 802.16m SRD [2], 802.16m SDD [4] and it follows the style and format guidelines in [3].

- References

[1] IEEE 802.16-2009, “IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

[2] IEEE 802.16m-07/002r9, “802.16m System Requirements”

[3] IEEE C802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[4] IEEE 802.16m-09/0034r2, “The Draft IEEE 802.16m System Description Document”

- Proposed text change in the P802.16m/D2

-------------------------------  Text Start  ---------------------------------------------------

- 15. Advanced Air Interface

- 15.3Physical layer

15.3.5.4.1 Pilot patterns

[Modify the text in line 37, page 305 as follows in 15.3.5.4.1]

Base pilot patterns used for DL data transmission with one data stream in dedicated and common pilot scenarios are shown in Figure 489, with the subcarrier index increasing from top to bottom and the OFDM symbol index increasing from left to right. Subfigure (a) and Subfigure (b) in Figure 489 show the pilot locations for pilot stream 1 and pilot stream 2, respectively.

Base pilot patterns used for two DL data streams in dedicated and common pilot scenarios are shown in Figure 490, with the subcarrier index increasing from top to bottom and the OFDM symbol index increasing from left to right. Subfigure (a) and Subfigure (b) in Figure 490 shows the pilot location for pilot stream 1 and pilot stream 2 in a PRU, respectively. The number on a pilot subcarrier indicates the pilot stream the pilot subcarrier corresponds to. The subcarriers marked as 'X' are null sub-carriers, on which no pilot or data is transmitted.

The interlaced pilot patterns are generated by cyclic shifting the base pilot patterns. The interlaced pilot patterns are used by different BSs for one and two streams. Interlaced pilot patterns for one stream is shown in Figure 491 and interlaced pilot patterns on stream 1 and stream 2 for two streams are shown in Figure 492and Figure 493, respectively. Each BS chooses one of the three pilot pattern sets (pilot pattern set 0, 1, and 2) as shown in Figure 491 and Figure 492. The index of the pilot pattern set used by a particular BS with Cell_ID = k is denoted by pk. The index of the pilot pattern set is determined by the Cell_ID according to the following equation:

pk =floor(k,256)

For one stream, each BS additionally chooses one of the two stream sets (stream set 0 and 1) within each pilot pattern set.

The index of the stream, denoted by sk, shall be determined according to the following equation:

sk=mod(k,2)

(a) Pilot pattern set 0 and stream set 0                     (b) Pilot pattern set 0 and stream set 1

(c) Pilot pattern set 1 and stream set 0                      (d) Pilot pattern set 1 and stream set 1

(e) Pilot pattern set 2 and stream set 0                    (f) Pilot pattern set 2 and stream set 1

Figure 491. Interlaced pilot patterns for 1 pilot stream

-------------------------------  Text End  ---------------------------------------------------