| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Length Field Description in MEH (section 15.2.2.2.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: MEH Format (Section 15.2.2.2.3) |  |
| Abstract | This contribution clarifies the length field description in MEH format |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Length Field Description in MEH (section 15.2.2.2.3)

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

Length field description in MEH format in D2 [1] is not clear. In table 661, how to determine the MAC PDU payload length is not clear.

# Proposed text

[Modify the table 661, page 23, as follows]

Table 661 – MEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEH () { |  |  |
| Last | 1 | 0 = another extended header follows MEH 1 = another extended header does not follow MEH |
| Type | TBD | MEH type |
| Num Flows | 4 | Number of Flow information present in the MEH. If ‘n’ connections are multiplexed, ‘n-1’ Flow IDs and Lengths are present. . |
| for (i = 1; i <= num flows; i++) { |  |  |
| Flow ID | 4 | Flow Identifier. The ‘i’th Flow ID indicates the Flow ID of the ‘i+1’th connection. |
| Length | 11 | Length of the connection payload. The ‘i’th length field indicates the length of the payload of the ‘i+1’th connection. The length of the payload of the first connection is given by “MAC PDU payload length (after decryption) – sum of ‘n-1’ Length fields”. where MAC PDU payload length (after decryption) = Length of MAC PDU (given by length field in GMH) - Length of GMH(2 bytes) - sum of length of all extended headers present in MAC PDU - Length of PN& EKS (3bytes if present) - Length of ICV ( 8 bytes, if present). |
| } |  |  |
| Reserved | variable |  |
| } |  |  |

# Reference

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”