| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Length Field Description in FPEH (section 15.2.2.2.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: FPEH Format (Section 15.2.2.2.1) |  |
| Abstract | This contribution clarifies the length field description in FPEH format |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Length Field Description in FPEH (section 15.2.2.2.1)

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

Length field description in FPEH format in D2 [1] is not clear. The length field description needs to be clarified for both multiplexing and non multiplexing case.

# Proposed text

[Modify the table 657, page 20, as follows]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FPEH() { |  |  |
| RI | 1 | Re-arrangement information indicator |
| SN | 10 | SN is maintained per connection. For non ARQ connection, ‘SN’ represents the MAC PDU Payload Sequence Number and the ‘SN’ value increments by one (modulo 1024) for each MAC PDU. For ARQ connection, ‘SN’ represents the ARQ block sequence number. |
| FC | 2 | Fragmentation Control bits (see Table 659) |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after FPEH |
| AFP | 1 | ARQ feedback poll indicator 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in GMH |
| if (RI == 1) { |  |  |
| LSI | 1 | Last ARQ sub-block indicator 0 = indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| SSN | TBD | SUB-SN of the first ARQ sub-block |
| } |  |  |
| Do { |  |  |
| End | 1 | Indication of more information 0 = Indicating another ‘Length’ and ‘End’ fields are followed 1 = Indicating no more ‘Length’ and ‘End’ fields are followed |
| if (End == 0) { |  |  |
| Length | 11 | MAC PDU with single connection payload: This field indicates the length of SDU or SDU fragment in the MAC PDU payload. If a the MAC PDU payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present in FPEH. These represent the length of first 'N-1' SDU/SDU fragments in MAC PDU payload. The length of the last SDU or SDU fragment in the MAC PDU payload is = MAC PDU payload length (after decryption) - sum of 'N-1' length fields in FPEH. where MAC PDU payload length (after decryption) = Length of MAC PDU (given by length field in GMH) - Length of GMH(2 bytes) - sum of length of all extended headers present in MAC PDU - Length of PN& EKS (3bytes, if present) - Length of ICV ( 8 bytes, if present) MAC PDU with multiple connections payload: MAC PDU payload consists of multiple connection payload. This field indicates the length of SDU or SDU fragment in the connection payload corresponding to this FPEH. If the connection payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present in FPEH. These represent the length of first 'N-1' SDU/SDU fragments in connection payload. The length of the last SDU or SDU fragment in the connection payload is = Connection payload length (see table 661, MEH format) - sum of 'N-1' length fields in FPEH. |
| } |  |  |
| } while (!End) |  |  |
| Reserved | Variable | Reserved bits are added at the end of FPEH for byte alignment. |
| } |  |  |

# Reference

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”