| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ACK Timer description (section 15.2.21) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: Management Message Reliability (Section 15.2.21) |  |
| Abstract | This contribution clarifies the ACK Timer description in Management Message Reliability |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ACK Timer description (section 15.2.21)

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

While sending the last fragment with polling bit set to '1' other fragments may already be under transmission. So if the ACK timer is started while sending the last fragment with polling bit set to '1', the ACK timer may expire even before transmission of all fragments is completed because of delay due to HARQ retransmissions and retransmission triggered by local nack.

The receiver responds with ACK after receiving all fragments of the control message. The receiver has time equal to value of ACK timer to respond with a ACK after receiving all fragments of the control message.

So the ack timer shall be started only after the transmission of control message or all fragments of contro message is completed.

# Proposed text

[Modify the lines 18-23 on page 258, section 15.2.21 as follows:]

The transmitter may request receiver to send a MAC layer acknowledgement to determine the status of the transmitted control message. When a MAC layer acknowledgement is used, the transmitter shall set the Polling bit to 1 in FEH of the MAC PDU containing the complete control message or the last pending fragment of the control message. and The transmitter shall start a ACK timer immediately after completing the transmission of unfragmented control message or all fragments of the control message, to wait for AAI_MSG_ACK message or Message ACK extended header.

# Reference

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”