| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for AMS scanning of neighbor Femto ABSs (15.4.8.1 Network Topology Acquisition) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jinyoung Chun, Inuk Jung, Kiseon Ryu, Bin-Chul Ihm LG Electronics | jychun03@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30a P802.16m/D2 : Support for Femto ABS (15.4) |  |
| Abstract | This contribution proposes the method of Femtocell BS detection. |  |
| Purpose | For member’s review and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for AMS scanning of neighbor Femto ABSs

Jinyoung Chun, Inuk Jung, Bin-Chul Ihm, and Kiseon Ryu

LG Electronics

- Introduction

In the section 15.4.8 Handover, an AMS may scan femto ABSs according to the FAs included in the broadcaset AAI_NBR-ADV message. If the AMS can know the accessible neighbor femto ABSs, the AMS can handover faster. In this contribution, we propose the method to get the list of the accessible neighbor Femto ABSs.

- Text proposal for inclusion in the 802.16m draft

Add the texts and figures marked in blue and remove the texts and figures marked in red in the P802.16m/D2.

[Insert the below text in line 36, page 549.]

--------------------------------------  Text Start  --------------------------------------------

15.4.8.1.2 AMS scanning of neighbor Femto ABSs

For neighbor Femto ABSs, an AMS performs the scanning procedure as per 15.2.6.1.2 with exceptions described in this subsection. An AMS may scan femto ABSs according to the FAs included in the broadcast AAI_NBR-ADV message. AMS may scan femto ABSs that are not included in AAI_NBR-ADV based on SA-preamble partitioning information (see 15.4.4). In addition, an AMS may scans femto ABSs based on the CSG White List. Based on location information, AMS may initiate the scanning procedure (See 15.2.6.1.2). The AMS may request additional scanning opportunity by sending AAI-SCN-REQ including the detected SA-preamble index.

For helping AMS’s scaning, the macro ABS may unicast a list of accessible neighboring Femto ABSs to the AMS. The femto ABS monitors the UL signals from the AMSs served by macro ABS from the pre-defined radio resource region. The UL signal may be the Tx power report header or PFBCH with the fixed Tx power, and the radio resource region is negotiated between Femto ABS and macro ABS over the backhaul. After monitoring, the femto ABS reports the measurement results to the macro ABS for identifying the handover opportunities from macrocell to Femtocell over the backhaul.

-----------------------------------------  Text End  ------------------------------------------