| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Change Proposal for DL MIMO control parameters table (15.3.7.2.5.5) |  |
| Date Submitted | 2009-11-xx |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics | wbong@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot 30a |  |
| Abstract | Text Change Proposal for DL MIMO control parameters table |  |
| Purpose | For member’s review and adoption into P802.16m_D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Change Proposal for DL MIMO control parameters table (15.3.7.2.5.5)

Wookbong Lee, Bin-Chul Ihm

LG Electronics

- Introduction

In this contribution, we modify current 802.16m draft standard (IEEE P802.16m/D2) so that DL control channel name can be consistently defined over several sections.

- References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”

- Proposed text change for 802.16m Multi-BS MIMO Amendment Text

Instruction to editor;

Black text: existing D2 text

Red and strike out text: for deletion

Blue and underline text: for addition

--------------------------------------------------------Text Start -----------------------------------------------------

15.3.7.2.5.5 Downlink signaling support of DL-MIMO modes

The BS shall send some parameters necessary for DL MIMO operation in a broadcast message. The broadcast information is carried in the S-SFH SP3 IE or in the additional broadcast information such as AAI_SCD or AAI_DL_IM messages.

The BS shall send some parameters necessary for DL MIMO operation in a unicast message. The unicast information is carried in the DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE, in the FBCH_Alloc_IEFeedback Allocation A-MAP IE, and Feedback Polling A-MAP IE, or in the Sounding_ IE.

Table 692832 specifies the DL control parameters required for MIMO operation.

Table 832 – DL MIMO control parameters

| Parameter | Description |  | Value | DL Control channel | Notes |
| --- | --- | --- | --- | --- | --- |
| Broadcast Information |  |  |  |  |  |
| Nt | Number of transmit antennas at the BS |  | 2, 4, 8 | SFH (system information) | Indicated in S-SFH (system information) SP3 IE |
| OL_Region | OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of streams (1 or 2) |  | OL-Region-ON (1 bit): Signal the existence of OL region. OL-Rank1-Config (3 bit): to signal the combination of sub-band and mini-band in OL region type 1. Refer to table 691. SB-OL-Region-2-Size (4 bit) : signal the number of sub-bands in OL region type 2. | Additional broadcast information | OL region signaling is SFH-SP3 or ABI (TBD) Indicated in AAI_SCD Message |
| BC_SI | Rank-1 base codebook subset indication for interference mitigation with PMI coordination |  | BitMAP: 8 bits if Nt=2 16 bits if Nt=4, 8 | Additional broadcast information | Rank-1 codebook element restriction/recommendation information It shall be ignored if CM = 0b00, 0b01 or 0b10 Indicated in AAI_DL_IM Message |
| Unicast Information |  |  |  |  |  |
| MEF | MIMO encoder format |  | SFBC Vertical encoding Horizontal encoding | A-MAP IE | MIMO encoder format. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| Mt | Number of streams in transmission |  | 1 to 8 |  | Number of streams in the transmission. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| SI | Index of allocated pilot stream |  | 1 to 4 |  | SI shall be indicated if MEF is HE Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| Feedback Allocation IEs |  |  |  |  |  |
| MFM | MIMO feedback mode | Refer to Table 691 |  |  | To decide the feedback content and related MS processing Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| MaxMt | Maximum number of streams | If MFM indicates a SU feedback mode: 1, 2, 4 or 8. If MFM indicates a MU feedback mode: 1, 2, 3, or 4. |  |  | If MFM indicates a SU feedback mode: the maximum number of streams scheduled for each user If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| CS indication | Codebook subset type for CL MIMO modes 2 and 4 | Base codebook or codebook subset |  |  | Depending on the MFM and CS indication, the MS shall feedback a PMI from the SU or MU base codebook, or from a subset of the SU or MU of the base codebook. Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| CM | Codebook feedback mode for CL MIMO modes 2 and 4 | 0b00: basestandard mode with codebook coordination disabled 0b01: transformation mode with codebook coordination disabled 0b10: differential mode with codebook coordination disabled 0b11: basestandard mode with codebook coordination enabled |  |  | In codebook-based feedback mode: Codebook coordination disabled: MS finds PMI within whole codebook type entry indicated by CS indication Codebook coordination enabled: MS shall find the rate-1 rank-1 PMI within codebook entries indicated by BC_SI and CS indication Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |

The indication of the type of MIMO feedback for CL MIMO modes 2 and 4 depends on the IE type sent by the BS. Fast feedback channel allocation IE indicates that the MS shall use codebook-based feedback. The sounding commend IE indicates that the MS shall use uplink sounding.

When CS indication indicates the use of a codebook subset and MFM indicates a SU CL SU MIMO mode(MFM = 3 and 4), the MS shall use the SU base codebook subset of Table 709849 when Nt=4. When CS indication indicates the use of a codebook subset and MFM indicates a MU CL MU MIMO mode(MFM = 6 and 7), the MS shall use the MU base codebook subset of Table 709849 when Nt =4.

--------------------------------------------------------Text End -----------------------------------------------------