| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Change Proposal for MAC control message section (15.2.3.33) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics David Mazzarese, Sangheon Kim Samsung Electronics | wbong@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot 30a |  |
| Abstract | Text change proposal for MAC control message section (MIMO feedback) |  |
| Purpose | For member’s review and adoption into P802.16m_D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Change Proposal for MAC control message section (15.2.3.33)

Wookbong Lee, Bin-Chul Ihm

LG Electronics

David Mazzarese, Sangheon Kim

Samsung Electronics

- Introduction

In this contribution, we provide editorial changes in 15.2.3.33 AAI_SingleBS_MIMO_FBK and 15.2.3.34 AAI_MultiBS_MIMO_FBK. Moreover, currently maximum 4 MS can be multiplexed, so the proper number of bits for subband stream index in MFM equals 5 is 1 or 2 and Additional measurement metric for Multi-BS MIMO has been decided in Jeju session.

- References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”

- Proposed text change

Instruction to editor;

Black text: existing D2 text

Blue and strike out text: for deletion

Red and underline text: for addition

--------------------------------------------------------Text Start -----------------------------------------------------

15.2.3.34 AAI_MultiBS_MIMO_FBK

The AAI_ MultiBS _MIMO_FBK message format is defined in Table 701. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting multi-BS MIMO feedback.

Table 701. AAI_MultiBS_MIMO_FBK message format

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| AAI_MultiBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type = xx |  |  |
| if (ICT ==0b10 or 0b11){ |  | Co-MIMO or CL-MD |
| CQI | 4 |  |
| } |  |  |
| for (i = 1; i <= N_multiBS_reports; i++) { |  |  |
| Temp_BSID | [TBD]4 | Diversity set member ID |
| PMI | 3 to 6 variable | PMI from the rank-1 base codebook or base codebook subset Nt=2: 3 bits Nt=4: 4 bits Nt=8: 4 bits |
| If (ICT==0b00 or 0b01) { |  |  |
| Additional measurement metric | [TBD]2 | [TBD]SINR gain assuming the reported PMI set is coordinated. This can be used for resolving conflict from multiple AMS. 0b00: 0.25dB 0b01: 0.50dB 0b10: 1.00dB 0b11: above 1.50dB |
| PMI_subset_size | 1 | Indication whether 1 PMI or a PMI set is feedback 0b0: 1 PMI 0b1: multiple PMIs |
| If (PMI_subset_size == 0b1){ |  |  |
| PMI_coordination_subset | 1 | Set of PMIs from the rank-1 base codebook or base codebook subset 0b0: correlation level n1 (As defined in section 15.5.1.1.1 cf Table 54) 0b1: correlation level n2 (As defined in section 15.5.1.1.1 cf Table 54) |
| } |  |  |
| } |  |  |
| If (ICT==0b10 or 0b11 ) { |  | ICT is defined in Feedback Polling A-MAP IE |
| If (CPI == 0b0){ |  | CPI is defined in Feedback Polling A-MAP IE |
| CPMI | [TBD]3 | Concatenating PMI for neighboring cells. |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

--------------------------------------------------------Text End -----------------------------------------------------