| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed change for Downlink FFR section (15.2.20.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm, HanGyu Cho LG Electronics | Voice: +82-31-450-1883 E-mail: wbong@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Provide a mechanism for FFR power boosting level |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed change for Downlink FFR section (15.2.20.1)

Wookbong Lee, Bin-Chul Ihm, HanGyu Cho

LG Electronics

- Introduction

In this contribution, we propose details of power boosting level for Downlink FFR operation. In addition to that we made some editorial changes in the DL FFR subsection.

- 2. Possible FFR boosting level

Details of power boosting level are not defined in current IEEE 802.16m draft standard (D2). In this contribution, we proposed one possible power boosting level detail.

As specified in table 771 to 773, the ratio of FFR partition resource size (FP1:FP2:FP3) is 1:1:1.

Assuming, as defined in D2, FP0’s power level is same as reference signal’s power level, which is 1 (0dB).

Then the possible combination of power boosting/de-boosting level is as in the following table.

| FP1 | FP2 | FP3 | FP1 | FP2 | FP3 |
| --- | --- | --- | --- | --- | --- |
| 4.8dB | -Inf | -Inf |  |  |  |
| 4.4dB | -6.0dB | -Inf | 4.0dB | -6.0dB | -6.0dB |
| 4.3dB | -5.0dB | -Inf | 3.7dB | -5.0dB | -5.0dB |
| 4.2dB | -4.0dB | -Inf | 3.4dB | -4.0dB | -4.0dB |
| 4.0dB | -3.0dB | -Inf | 3.0dB | -3.0dB | -3.0dB |
| 3.7dB | -2.0dB | -Inf | 2.4dB | -2.0dB | -2.0dB |
| 3.4dB | -1.0dB | -Inf | 2.0dB | -1.5dB | -1.5dB |
| 3.0dB | 0.0dB | -Inf | 1.5dB | -1.0dB | -1.0dB |
| 2.0dB | 1.5dB | -Inf | 1.0dB | -0.6dB | -0.6dB |
| 1.0dB | 2.4dB | -Inf | 0.0dB | 0.0dB | 0.0dB |

Here is summary of possible boosting level for FP1; 4bit: {0.0, 1.0, 1.5, 2.0, 2.4, 3.0, 3.4, 3.7, 4.0, 4.2, 4.3, 4.4, 4.8, reserved, reserved, reserved}

And here is summary of possible boosting level for FP2 or FP3; 4bit:{-Inf, -6.0, -5.0, -4.0, -3.0, -2.0, -1.5, -1.0, -0.6, 0.0, 1.5, 2.4, reserved, reserved, reserved, reserved}

- 3. Proposed text change in the 802.16m amendment

--------------------------------------------------------------------- Text Start --------------------------------------------------------------------------------

Instruction to editor;

Black text: existing D2 text

Red and strike out text: for deletion

Blue and underline text: for addition

Remedy #1 Replace figure 457 and modify text below figure as follows;

Enlarge the heights of the block in Cell 2 of partition (2) and Cell 3 of partition (3) as same as the block heights in Cell 1 of partition 1. And insert texts as follows;

Figure 457—Basic concept of fractional frequency reuse

Remedy #2 Line 6, page 251

Modify text as follows;

Cell k = floor(IDcell/256)+1

Remedy #3 Table 757

Modify the table as follows;

Each frequency partition’s power differences relative to the reference power level are broadcasted in ABIAAI_DL_IM message as shown in the Table xx757.

Table 757 - FFR partition power boosting/de-boosting level for cell 1

| Syntex | Size (bits) | Notes |
| --- | --- | --- |
| FP1_power | [4] | 0b0000: 0.0dB, 0b0001: 1.0dB, 0b0010: 1.5dB, 0b0011: 2.0dB, 0b0100: 2.4dB, 0b0101: 3.0dB, 0b0110: 3.4dB, 0b0111: 3.7dB, 0b1000: 4.0dB, 0b1001: 4.2dB, 0b1010: 4.3dB, 0b1011: 4.4dB 0b1100: 4.8dB, 0b1101~0b1111: reserved xdB bBoosted over reference signal |
| FP2_power | [4] | 0b0000: -Inf, 0b0001: -6.0dB, 0b0010: -5.0dB, 0b0011: -4.0dB, 0b0100: -3.0dB, 0b0101: -2.0dB, 0b0110: -1.5dB, 0b0111: -1.0dB, 0b1000: -0.6dB, 0b1001: 0.0dB, 0b1010: 1.5dB, 0b1011: 2.4dB 0b1100~0b1111: reserved xdB bBoosted over reference signal |
| FP3_power | [4] | 0b0000: -Inf, 0b0001: -6.0dB, 0b0010: -5.0dB, 0b0011: -4.0dB, 0b0100: -3.0dB, 0b0101: -2.0dB, 0b0110: -1.5dB, 0b0111: -1.0dB, 0b1000: -0.6dB, 0b1001: 0.0dB, 0b1010: 1.5dB, 0b1011: 2.4dB 0b1100~0b1111: reserved xdB bBoosted over reference signal |

* Note that –Inf means, ABS sends no data for the frequency partition.

Remedy #4 Line 16-43, page 78

Modify text as follows;

15.2.3.30 AAI_DL_IM Message

An AAI_DL_IM broadcast DL FFR partition resource metric, BC_SI, NIP_threshold, power level for each frequency partition.

-- ASN1START

AAI_DL_IM Message ::= SEQUENCE {

FFR partition resource metric :: = SEQUENCE {

Resource_Metric_FP2			INTEGER (TBD) OPTIONAL

Resource_Metric_FP3			INTEGER (TBD) OPTIONAL

}

BC_SI						INTEGER (0..15) OPTIONAL

NIP_th_1					INTEGER (TBD) OPTIONAL

NIP_th_2					INTEGER (TBD) OPTIONAL

FP1_power					INTEGER (TBD0..15) OPTIONAL

FP2_power					INTEGER (TBD0..15) OPTIONAL

FP3_power					INTEGER (TBD0..15) OPTIONAL

}

-- ASN1STOP

--------------------------------------------------------------------- Text End --------------------------------------------------------------------------------