| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Change Proposal for Extended header section (15.2.2.2.7) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics | wbong@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot 30a |  |
| Abstract | Text change proposal for Extended header section (MIMO feedback) |  |
| Purpose | For member’s review and adoption into P802.16m_D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Change Proposal for Extended header section (15.2.2.2.7)

Wookbong Lee, Bin-Chul Ihm

LG Electronics

- Introduction

In this contribution, we provide editorial changes in 15.2.2.2.7 Correlation Matrix Feedback Extended Header (CMFEH) and 15.2.2.2.8 MIMO feedback extended header (MFEH). Moreover, currently maximum 4 MS can be multiplexed, so the proper number of bits for subband stream index in MFM equals 5 is 1 or 2.

- References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”

- Proposed text change

Instruction to editor;

Black text: existing D2 text

Red and strike out text: for deletion

Blue and underline text: for addition

--------------------------------------------------------Text Start -----------------------------------------------------

15.2.2.2.7 Correlation Matrix Feedback Extended Header (CMFEH)

This Correlation matrix feedback extended header format is defined in Table 665. This header may be used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the quantized transmit correlation matrix when the ABS is equipped with 2 or 4 transmit antennas.

Table 665 - Correlation matrix feedback extended header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CMFEH() { |  |  |
| LAST | 1 | Set to “1” if last EH, else, set to “0” |
| Type | TBD | CMFEH type |
| For (i=1; i ≤ Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in <<Table 713850>> |
| For (j=i+1; j ≤ Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in <<Table 714851>> |
| } |  |  |
| } |  |  |
| } |  |  |

15.2.2.2.8 MIMO feedback extended header (MFEH)

This MIMO feedback extended header format is defined in Table 666. This header is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for one the best subband for MFM 2, 3, 5 or 6.

Table 666 – MIMO feedback extended header formatCorrelation matrix feedback extended header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFEH() { |  |  |
| LAST | 1 | Set to “1” if last EH, else, set to “0” |
| Type | TBD | MFEH type |
| MFM | 3 |  |
| If (MFM == 0 or 1){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 1 to 2 | Depending on MaxMt |
| } |  |  |
| If (MFM == 4 or 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | 3 to 5 | Depending on KSB bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If (MFM == 3){ |  |  |
| Subband index | 3 to 5 | Depending on KSB bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| Subband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| If (MFM == 5){ |  |  |
| Subband index | 3 to 5 | Depending on KSB bandwidth |
| Subband CQI | 4 |  |
| Subband stream index | 1 to 32 | Depending on MaxMt |
| } |  |  |
| If (MFM == 6){ |  |  |
| Subband index | 3 to 5 | Depending on KSB bandwidth |
| Subband CQI | 4 |  |
| Subband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| } |  |  |

--------------------------------------------------------Text End -----------------------------------------------------