| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Change Proposal on DL Subchannelization for Supporting OL Region (15.3.5) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Wookbong Lee, HanGyu Cho, Bin-Chul Ihm LG Electronics | wbong@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot 30a |  |
| Abstract | Text Change Proposal for DL MIMO control parameters table |  |
| Purpose | For member’s review and adoption into P802.16m_D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Change Proposal on DL subchannelization for Supporting OL Region (15.3.5)

Wookbong Lee, HanGyu Cho, Bin-Chul Ihm

LG Electronics

- Introduction

In this contribution, we modify current 802.16m draft standard (IEEE P802.16m/D2) so that we can operate OL Region correctly.

- Motivation/Rationale

An open-loop region is defined as a time-frequency resource using same number of pilot pattern and a given open-loop MIMO mode. The open-loop region allows base stations to coordinate their open-loop MIMO transmissions, in order to offer a stable interference environment where the precoders and number of MIMO streams are not time-varying.

Currently there are three kinds of open-loop region, OL region type 0, OL region type 1, and OL region type 2.

The following table shows details of OL region.

Types of open-loop regions

|  | MaxMt | MIMO Mode | MIMO feedback mode | Supported permutation |
| --- | --- | --- | --- | --- |
| OL Region type 0 | 2 streams | MIMO mode 0 MIMO mode 1 (Mt = 2 streams) | 0 | DRU |
| OL Region type 1 | 1 streams | MIMO mode 5 (Mt = 1 stream) | 1 | Miniband based CRU (diversity allocation) |
|  |  |  | 2 | Subband based CRU (localized allocation) |
| OL Region type 2 | 2 streams | MIMO mode 1 (Mt = 2 streams) MIMO mode 3 (Mt = 2 streams) | 5 | Subband based CRU (localized allocation) |

To enable the open-loop region, we need to make sure the resource units assigned for OL region shall be aligned across cells.

However, current subchannelization rule can’t make the OL region alignment across cell.

This problem happens when different cell has different number of subbands, minibands, or DRU.

Moreover, even though each cell has same number of subbands, minibands and DRU, the location of minibands and DRUs are not aligned due to sector/cell specific CRU/DRU allocation sequence applied for miniband and DRU mapping. (See equation (197) for more details)

We propose to disable the sector/cell specific CRU/DRU allocation sequence for miniband and DRU mapping or using the same SEED value among cells for the sequence generation when OL Region type 0, OL Region type 1 miniband mode, or OL Region type 1 miniband and subband mix mode.

Proposal:

[Insert the following sentence in line xx of page yy.] In order to support OL Region type 0, OL Region type 1 miniband mode, or OL Region type 1 miniband and subband mix mode, each cell/sector, who are using the same OL Region, broadcasts the same value of DCASSB,i, DCASMBi, DCASi.

From

SEED = {IDcell*343}mod 210

DRUFPi[j] = PRUFPi[k+LSB-CRU,FPi], 0≤j<LDRU,FPi

Where k = s[j+LCRU,FPi – LSB-CRU,FPi], s[j] = {PermSeq(j)+DL_PermBase} mode(FPSi – LSB-CRU,FPi)

To

[Insert the following sentence in line xx of page 197.] In order to support OL Region type 0, OL Region type 1 miniband mode, or OL Region type 1 miniband and subband mix mode, one of the following options are possible.

Option 1. SEED = {PermBase_OLRegion*343}mod 210, where PermBase_OLRegion is a fixed value which is common for all cell/sectors (e.g., PermBase_OLRegion=0) or broadcasted by SFH SP1.

Option 2. DRUFPi[j] = PRUFPi[j+LSB-CRU,FPi], 0≤j<LDRU,FPi

In option 1, OL Region related parameter for type 0 and 1 shall be transmitted via SFH SP1 as well.

In option 2, OL Region related parameter for type 0 and 1 shall be transmitted via SFH SP1 as well unless we need disable the permutation all the time.

- References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”

- Proposed text change

Instruction to editor;

Black text: existing D2 text

Red and strike out text: for deletion

Blue and underline text: for addition

--------------------------------------------------------Text Start -----------------------------------------------------

--------------------------------------------------------Text End -----------------------------------------------------